Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 50 Science Pk 2 | Relocation of engine | 1 | Lift up engine using crowbar |  |
|  |  |  | 2 | Insert pallet truck under engine bed |  |
|  |  |  | 3 | Transport engine to new location within 2m away |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of engine | RA Member: Zailani |  |  |  |
| Location: | 50 Science Pk 2 #04-10 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 30/06/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up engine using crowbar | Engine topple due to lost of balance upon lifted up | Bodily injury & damage to property | - Workers to be alert when lift up engine - Ensure engine are lift up at the right angle of gravity - Lift up engine to 3” high from one side to another is carried out in stages with the use of hard woodblock to insert under bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 01/07/21 |  |
| 2 | Insert pallet truck under engine bed | Sharp edges | Hand injury | - Keep hand/finger clear of sharp edges - Insert pallet truck to below engine bed with care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 01/07/21 |  |
| 3 | Transport engine to new location within 2m away | Unstable load during moving | Bodily injury & damage to property if equipment topper | - Ensure pallet truck are place accordingly on engine bed - Ensure engine load are spread evenly and balance well on the pallet ruck before being transfer to new location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 01/07/21 |  |
|  |  | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity pallet truck to reduce the effort of each man when pushing the engine - Supervisor to ensure there are adequate manpower to be deploy for the job | 3 | 3 | 9 | Use manual puller if needed to assist in pulling the engine | 3 | 2 | 6 | Supervisor on site | 01/07/21 |  |
|  |  | Trip & fall | Sprain & strain | - Working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Any oil or water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 01/07/21 |  |
|  |  | Obstruction to vision while moving | Damage to property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of engine so as to have a clearer view before hitting any other obstacles | 2 | 1 | 2 | Supervisor on site | 01/07/21 |  |