Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | NUS | Relocation of machine | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Insert rollers/pallet truck |  |
|  |  |  | 3 | Transport machine to new location within lab |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of machine | RA Member: Zailani |  |  |  |
| Location: | NUS, 4 Engineering Dr 3 Blk E4 #02-06 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 19/07/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topple due to lost of balance upon lifted up | Multiple injuries & damage to property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of solid woodblock to be insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel - PPE (safety shoe, glove) provided and put on | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 21/07/22 |  |
| 2 | Insert rollers / pallet truck | Sharp edges | Minor cuts or bruises | - Keep hand/finger clear of sharp edges - Insert rollers to below machine bed with care - PPE (safety shoe, glove) provided and put on - Remove woodblock only when rollers/pallet truck are properly secured onto machine bed | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 21/07/22 |  |
| 3 | Transport machine to new location within lab | Unstable load during moving or turning | Multiple injuries & damage to property if machine topper | - Ensure rollers/pallet truck are place accordingly on machine bed - Ensure rollers are properly secured onto machine bed with bolts and nuts - Ensure machine load are spread evenly and balanced well on the rollers/pallet truck before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 21/07/22 |  |
|  |  | Excessive weight load | Sprain back if force moving the load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Buddy system-one to control the pallet truck steering while the others to assist in pushing the machine - Supervisor to ensure there are adequate manpower to be deploy for the job - Ensure travelling ground is protected with metal sheet in contact with the rollers at all time when required | 3 | 3 | 9 | Use of manual puller to assist in pulling the machine when required | 3 | 2 | 6 | Supervisor on site | 21/07/22 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 3 | 1 | 3 | Supervisor on site | 21/07/22 |  |
|  |  | Obstruction to vision while moving | Damage to property | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers/pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of equipment so as to have a clearer view before hitting any other obstacles or people | 1 | 2 | 2 | Supervisor on site | 21/07/22 |  |