Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 18 Bedok South Rd | Relocation of machine | 1 | Jack up machine using manual/hydraulic jack |  |
|  |  |  | 2 | Insert rollers |  |
|  |  |  | 3 | Transportation of machine to new location within production |  |
|  |  |  | 4 | Covid-19 |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of machine | RA Member: Zailani |  |  |  |
| Location: | 18 Bedok South Rd Hamilton Sundstrand Pacific Collins Aerospace Systems | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 01/06/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine using manual/ hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily injury & damage to property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard wooden block - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | TBC |  |
| 2 | Insert rollers | Sharp edges | Hand injury | - Keep hand/finger clear of sharp edges - Insert rollers to below machine base with care - Use glove - Remove woodblock only when rollers are properly secured onto machine base | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | TBC |  |
| 3 | Transportation of machine to new location within production | Unstable load during moving or turning | Bodily injury & damage to property when machine fall | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured onto machine base with bolts and nuts - Ensure machine load are spread evenly and balanced well on the rollers before being transport to designated new location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | TBC |  |
|  |  | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate manpower to be deploy for the job - Ensure travelling ground is protected with canvas/metal plate in contact with the rollers at all time | 3 | 3 | 9 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 2 | 6 | Supervisor on site | TBC |  |
|  |  | Passageway block by materials and tooling | Trip & fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | TBC |  |
|  |  | Obstruction to vision while moving | Damage to property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | TBC |  |
|  | Covid-19 | Spread of Covid-19 | Staff Visitors | - Hand washing facilities with soap and water in place | 2 | 2 | 4 | Staff/visitors to be reminded on regular basis to wash their hand for 20 seconds with water and soap To avoid touching face, eyes, nose or mouth with unclean hand | 2 | 2 | 4 | Supervisor on site | Immediate |  |
|  |  |  |  | - Ensure temperature taking twice daily to all personnel, including staff, visitors and suppliers when entering into company premise | 2 | 2 | 4 | Check to be carried by manager to ensure that necessary procedure are being followed | 2 | 2 | 4 | Supervisor on site | Immediate |  |
|  |  |  |  | - Workers are advised to try practicing social distancing of at least 1m in between persons at workplace premises when possible - To ensure that all onsite personnel, including staff, visitors and suppliers to wear a mask at all time when at workplace | 2 | 2 | 4 | Staff, visitors to be reminded on daily basis of the importance of social distancing both in the workplace and outside of it | 2 | 2 | 4 | Supervisor on site | Immediate |  |
|  |  |  |  | - Safe Entry System must be use to record track of all personnel, including staff, visitors and suppliers that check-in and check-out of workplace to help MOH in establishing potential transmission chains | 2 | 2 | 4 | In-house Safe Management Officer (SMO) to assist in the implementation, coordination and monitoring of the safe management measures at the workplace | 2 | 2 | 4 | Supervisor on site | Immediate |  |