Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 14 Gul Drive | Relocate machine within factory premise | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting operation – to hoist down forklift from lorry chassis |  |
|  |  |  | 4 | Jack up machine using hydraulic jack |  |
|  |  |  | 5 | Insert rollers under machine bed |  |
|  |  |  | 6 | Transport machine to new location within factory premise |  |
|  |  |  | 7 | Cutting of stud using hand grinder |  |
|  |  |  | 8 | Operation of electric hand drill |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocate machine within factory premise | RA Member: Zailani |  |  |  |
| Location: | 14 Gul Drive Premier Hytemp | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 23/12/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle to be move into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving vehicle - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | TBC |  |
| 2 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outrigger (must be able to see all operation procedure) - Ensure all outrigger beams to be fully extended and resting on steel plate/timber mats as base plate - PPE (safety shoe, safety helmet, glove) provided and put on | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel or vehicle near moving part of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | TBC |  |
| 3 | Lifting operation – to hoist down forklift from lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load - No defective gears allowed - Never stand close or under the load - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be on site to supervised the lifting operation | 4 | 1 | 4 | Lifting supervisor on site | TBC |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of lifting plan to all lifting personnel on site to ensure that lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | TBC |  |
|  |  | Struck by object due to unsafe slinging or rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgement - Ensure load is rig at the centre of gravity - Rigger to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 4 | 1 | 4 | Lifting supervisor on site | TBC |  |
| 4 | Jack up machine using hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine from one side to another is carried out in stages with the use of hard woodblock to insert under machine bed - Ensure communication between worker is clear - Supervisor to ensure operation area is clear of unauthorised personnel - PPE (safety helmet, safety shoe, glove) provided and put on | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | TBC |  |
| 5 | Insert rollers under machine bed | Sharp edges of machine | Minor cuts or bruises | - Keep hand/finger clear of sharp edges - Insert rollers to below machine bed with care - PPE (safety glove) provided and put on | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | TBC |  |
|  |  | Pinch point | Injury to hand | - Always keep your eye on your hand - Never place hand where you can’t see them - Ensure to have a clear view of what you are working on - If view is obstructed, do not proceed with the work until having a clearer view - To standby woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | TBC |  |
| 6 | Transport machine to new location within factory premise | Unstable load during moving or turning | Bodily Injury & Damage to property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured onto machine bed with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balanced well on the rollers before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed Ensure floor is properly sweep and clear of any debris on the floor in case rollers got stuck by the debris | 4 | 1 | 4 | Supervisor on site | TBC |  |
|  |  | Excessive weight load | Sprain back if force moving heavy load | - Use the correct capacity rollers for the move - Proper tools such as manual puller to be brough to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt if needed to prevent sprain back or back ache | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in pushing/ pulling the machine when required | 3 | 1 | 3 | Supervisor on site | TBC |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 3 | 1 | 3 | Supervisor on site | TBC |  |
|  |  | Obstruction to vision while moving | Damage to property | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any other obstacles or people | 1 | 2 | 2 | Supervisor on site | TBC |  |
| 7 | Cutting of stud using hand grinder | Unsuitable tool use | Bodily Injury | - Supervisor to ensure tools provided to worker are fit for the purpose - Hand grinder use must be fitted with its proper guard as supplied by the manufacturer - Ensure hand grinder handle is in good condition for worker to have a firm grip | 3 | 2 | 6 | Worn out or damage tool must not be use and to be disposed | 3 | 1 | 3 | Supervisor on site | TBC |  |
|  |  | Spark from cutting stud | Bodily Injury & Damage to Property | - To obtain HWP from the host prior to commencement of work - To remove any combustible materials away from the work area - Use of long sleeve shirt - PPE (glove, safety goggle) provided and shall be worn by personnel operating the hand grinder while cutting the stud - Fire blanket will be placed near the work area - Supervisor to ensure no unauthorised personnel is nearby and to stay clear of work area | 3 | 2 | 6 | Regular enforcement on safety rules and regulation on site by supervisor | 3 | 1 | 3 | Supervisor on site | TBC |  |
| 8 | Operation of electric hand drill | Contact with drill bit | Hand Injury | - Brief workers on safe handling technique - Ensure drill bits are tightly secured and sharp and appropriate for the type of material being worked on - Remove any adjusting key or wrench before turning the power tool on - Ensure body parts or long hair are properly tied and keep clear from moving parts of the drill - Do not wear loose clothing or jewellery during operation - To ensure worker doing the drilling are competent in using tool issued to them | 3 | 2 | 6 | Stay alert and watch what you are doing Do not operate a power tool while you are tired of under medication | 3 | 1 | 3 | Supervisor on site | TBC |  |
|  |  | Exposed to dust | Ill Health | - PPE (mask, safety goggle) provided and shall be worn by personnel operating the hand drill to avoid inhaling excessive amount of dust formed while drilling - Use of vacuum pump to suck in all dust from drilling work | 3 | 2 | 6 | Close supervision and regular reminder by supervisor | 3 | 1 | 3 | Supervisor on site | TBC |  |
|  |  | Contact with exposed wiring | Electrocution | - Workers to visually check and leads for signs of damage to wiring and equipment before operation - All tools issue and use should be visually inspected for faulty wire - Worker to inform supervisor if wiring or tools use is found to be defective - Ensure no joint in extension wire use - Always ensure the extension cord has the right plug to accommodate all your power tools - Isolate power by removing from power source prior to performing any work on the equipment, including changing of bits | 4 | 2 | 8 | To keep cord away from heat, oil and sharp edges Never yank the cord to disconnect it from the receptacle | 4 | 1 | 4 | Supervisor on site | TBC |  |