Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 149 Gul Circle | Relocate machine within factory premise | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Jack up machine using hydraulic jack |  |
|  |  |  | 3 | Insert rollers under machine bed |  |
|  |  |  | 4 | Transport machine to new location within factory premise |  |
|  |  |  | 5 | Cutting of stud using hand grinder |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocate machine within factory premise | RA Member: Zailani |  |  |  |
| Location: | 149 Gul Circle FMC Technologies Singapore Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 13/06/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle to be move into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving vehicle - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 22-24 /07/24 |  |
| 2 | Jack up machine using hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine from one side to another is carried out in stages with the use of hard woodblock to insert under machine bed - Ensure communication between worker is clear - Supervisor to ensure operation area is clear of unauthorised personnel - PPE (safety helmet, safety shoe, glove) provided and put on | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 22-24 /07/24 |  |
| 3 | Insert electrical rollers under machine bed | Sharp edges of machine | Minor cuts or bruises | - Keep hand/finger clear of sharp edges - Insert rollers to below machine bed with care - PPE (safety glove) provided and put on | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 22-24 /07/24 |  |
|  |  | Pinch point | Injury to hand | - Always keep your eye on your hand - Never place hand where you can’t see them - Ensure to have a clear view of what you are working on - If view is obstructed, do not proceed with the work until having a clearer view - To standby woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 22-24 /07/24 |  |
| 4 | Transport machine to new location within factory premise | Unstable load during moving or turning | Bodily Injury & Damage to property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured onto machine bed with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balanced well on the rollers before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed Ensure floor is properly sweep and clear of any debris on the floor in case rollers got stuck by the debris | 4 | 1 | 4 | Supervisor on site | 22-24 /07/24 |  |
|  |  | Laying of canvas / metal sheet onto flooring | Cuts from sharp edges | - To replace defective sheet with a new one - PPE (glove) to be provided and put on carrying metal sheet - Buddy system-two worker to lift one piece of metal sheet and lay out onto flooring at a time | 2 | 2 | 4 | Regular check by storeman and ensure equipment issue is in good condition | 2 | 1 | 2 | Supervisor on site | 22-24 /07/24 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 3 | 1 | 3 | Supervisor on site | 22-24 /07/24 |  |
|  |  | Obstruction to vision while moving | Damage to property | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any other obstacles or people | 1 | 2 | 2 | Supervisor on site | 22-24 /07/24 |  |
| 5 | Cutting of stud using hand grinder | Unsuitable tool use | Bodily Injury | - Supervisor to ensure tools provided to worker are fit for the purpose - Hand grinder use must be fitted with its proper guard as supplied by the manufacturer - Ensure hand grinder handle is in good condition for worker to have a firm grip | 3 | 2 | 6 | Worn out or damage tool must not be use and to be disposed | 3 | 1 | 3 | Supervisor on site | 22-24 /07/24 |  |
|  |  | Spark from cutting stud | Bodily Injury & Damage to Property | - To obtain HWP from the host prior to commencement of work - To remove any combustible materials away from the work area - Use of long sleeve shirt - PPE (glove, safety goggle) provided and shall be worn by personnel operating the hand grinder while cutting the stud - Fire blanket will be placed near the work area - Supervisor to ensure no unauthorised personnel is nearby and to stay clear of work area | 3 | 2 | 6 | Regular enforcement on safety rules and regulation on site | 3 | 1 | 3 | Supervisor on site | 22-24 /07/24 |  |