Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 8 Tractor Rd | Relocation of machine | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Insert pallet truck |  |
|  |  |  | 3 | Transportation of machine to new location |  |
|  |  |  | 4 | Lift up granite table using battery lifter |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of machine | RA Member: Zailani |  |  |  |
| Location: | No 8 Tractor Rd Qioptiq Singapore Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 17/09/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 19/09/18 |  |
| 2 | Insert pallet truck | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert pallet truck to below machine base with care - Use of suitable hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 19/09/18 |  |
| 3 | Transportation of machine to new location | Unstable load during moving or turning | Bodily Injury & Damage to Property when machine topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and is stable on the pallet truck before being transport to designated location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 19/09/18 |  |
|  |  | Excessive weight load | Sprain Back when | - Use the correct capacity pallet truck to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system-one to control the pallet truck steering while the others to push the machine | 3 | 3 | 9 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 2 | 6 | Supervisor on site | 19/09/18 |  |
|  |  | Passageway block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 19/09/18 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before it hit any obstacles | 2 | 1 | 2 | Supervisor on site | 19/09/18 |  |
| 4 | Lift up granite table using battery lifter | Overload | Damage to Property | - Load weight should be known - To ensure granite table to be lift up is within the lifting capacity of the battery lifter | 3 | 2 | 6 | While lift up the granite table, try not to jerk the load | 3 | 1 | 3 | Supervisor on site | 19/09/18 |  |
|  |  | Unsafe rigging | Damage to Property | - Load must be securely slug and properly balanced to prevent from dislodgment - Ensure load is lift up at the centre of gravity - Lift up granite table using webbing sling to prevent damage on granite table | 3 | 2 | 6 | Ensure no unauthorised personnel within the working area | 3 | 1 | 3 | Supervisor on site | 19/09/18 |  |