Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 5 Tuas Drive 2 | Relocation of machine | 1 | Lift up granite table using battery lifter |  |
|  |  |  | 2 | Transportation of machine to new location |  |
|  |  |  | 3 | Lift up granite table to install back machine stand |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of Discovery II D8 CMM machine | RA Member: Zailani |  |  |  |
| Location: | No 5 Tuas Drive 2 Turbine Overhaul Services Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 02/12/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up granite table from stand using battery lifter | Falling load | Bodily Injury & Damage to Property | - Known the CG of the load - To ensure granite table is secured firmly and balanced well on the lifter fork - Lift up granite table at slow speed - To avoid sudden jerk of the load while being lifted - Ensure no unauthorised personnel is near or within the working area | 4 | 2 | 8 | Close supervision by supervisor to ensure all control and precaution are in place | 4 | 1 | 4 | Supervisor on site | 05/12/19 |  |
|  |  | Overloading of load | Damage to Property | - Load weigh should be known - Ensure granite table to be lifted up is within the lifting capacity of the battery lifter | 3 | 2 | 6 | While lift up the granite table, try not to jerk the load | 3 | 1 | 3 | Supervisor on site | 05/12/19 |  |
| 2 | Transportation of machine to new location | Unstable load during moving or turning | Bodily Injury & Damage to Property when machine topper | - Ensure pallet truck are place accordingly on pallet base - Ensure machine load are spread evenly and balanced well on the pallet before being transport to new designated location - Workers to be alert and keep close lookout on route of travel from point to point - Buddy system-one to control the pallet truck steering while the others to push and hold onto the machine | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 05/12/19 |  |
|  |  | Passageway block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 05/12/19 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 05/12/19 |  |
| 3 | Lift up granite table to install back machine stand | Falling load | Bodily Injury & Damage to Property | - Known the CG of the load - To ensure granite table is secured firmly and balanced well on the lifter fork - Lift up granite table at slow speed - To avoid sudden jerk of the load while being lifted - Ensure no unauthorised personnel is near or within the working area | 4 | 2 | 8 | Close supervision by supervisor to ensure all control and precaution are in place | 4 | 1 | 4 | Supervisor on site | 05/12/19 |  |