Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 15 Chin Bee Drive | Relocation of vehicle | 1 | Unloading of equipment from lorry chassis using forklift |  |
|  |  |  | 2 | Jack up vehicle using hydraulic jack |  |
|  |  |  | 3 | Insert rollers and I-beam onto vehicle base |  |
|  |  |  | 4 | Using rollers to transport vehicle to outside open space by Eric Wong personnel |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of vehicle | RA Member: Zailani |  |  |  |
| Location: | 15 Chin Bee Drive ST Engineering Land Systems Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 27/04/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Unloading of equipment from lorry chassis using forklift | Falling load | Multiple injuries & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load when being fork up - Ensure load are fork up at the centre of gravity and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - No jumping down from lorry when unloading/ loading - Operator advised to be careful when moving forward or reversing - Ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Use of extension fork if required to give greater stability of load | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 2 | Jack up vehicle using hydraulic jack | Vehicle topple due to lost of balance upon lifted up | Multiple injuries & damage to property | - Supervisor to pre brief all workers on job scope before commencement of work - Workers to be alert when jack up the vehicle - Ensure vehicle are jack up at the right angle of gravity - Jack up vehicle base from one side to another is carried out in stages - Ensured all tools & equipments use are according to work manual - Supervisor to ensure no one to be directly under the vehicle when its off load - Ensure no one is working on top or inside vehicle when jacking - Ensure floor is check for uneven or slippery conditions - Ensure communication is clear, if unsure to clarify - Supervisor to ensure operation area is clear of unauthorised personnel - Barricade off work area with cones and red/white tape - PPE (safety helmet, safety shoe, cotton glove) provided and put on | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 3 | Insert rollers and I-beam onto vehicle base | Sharp edges | Minor cuts or bruises | - Keep hand/finger clear of sharp edges - Insert rollers and I-beam to below vehicle base with care - PPE (safety helmet, safety shoe, cotton glove) provided and put on - Wheel-choke the rollers before jacking up the others side of vehicle - Avoid using hands to adjust or meddle if necessary | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | TBA |  |
| 4 | Using rollers to transport vehicle to outside open space by Eric Wong personnel | Unstable load during moving or turning | Multiple injuries & damage to property if equipment topper | - Ensure I-beam and electrical rollers are place accordingly on vehicle base - Ensure I-beam and rollers are properly secured onto vehicle base with bolts and nuts - Tighten bolts and nuts onto rollers with adjustable spanner - Ensure vehicle load are spread evenly and balanced well on the rollers before being transport to new location - Barricade off work area with cones and red/white tape - Ensure road condition is clear from traffic - Ensure building opening and height is acceptable for vehicle to be transport out - No one to stand directly in front or back of vehicle when in operation - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow transportation speed Bankman to be responsible in directing movement of vehicle where he can see and be seen by incoming vehicle | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping - Barricade with warning sign for power cord cable on the ground | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling within the working area | 3 | 1 | 3 | Supervisor on site | TBA |  |
|  |  | Obstruction to vision while moving | Damage to property | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the electrical rollers steering controller if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of equipment so as to have a clearer view before hitting any other obstacles or people | 1 | 2 | 2 | Supervisor on site | TBA |  |