Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 50 Nanyang Ave | Relocation of laser table | 1 | Lift up laser table using battery lifter |  |
|  |  |  | 2 | Tilt up laser table |  |
|  |  |  | 3 | Transportation of machine to new location |  |
|  |  |  | 4 | Lift up granite table to install back air coating |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of New Pot RX3000 Laser Table | RA Member: Zailani |  |  |  |
| Location: | NTU, 50 Nanyang Ave, Blk N3.2-B2-01 School of Mechanical & Aerospace Engineering | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 11/12/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up laser table from stand using battery lifter | Falling load | Bodily Injury & Damage to Property | - Known the CG of the load - To ensure laser table is secured firmly and balanced well on the lifter fork - Lift up laser table at slow speed - To avoid sudden jerk of the load while being lifted - Ensure no unauthorised personnel is near or within the working area | 4 | 2 | 8 | Close supervision by supervisor to ensure all precaution are in place | 4 | 1 | 4 | Supervisor on site | 17/12/19 |  |
|  |  | Overloading of load | Damage to Property | - Load weigh should be known - Ensure laser table to be lifted up is within the lifting capacity of the battery lifter | 3 | 2 | 6 | While lift up the granite table, try not to jerk the load | 3 | 1 | 3 | Supervisor on site | 17/12/19 |  |
| 2 | Tilt up laser table and load onto wooden base | Failing of lifting equipment | Bodily Injury & Damage to Property | - Ensure there is enough space for work to be done - To ensure lifting gears use are in compliance with weight of load - No lifting beyond SWL of equipment - Tilt up table at slow speed - To ensure no unauthorised personnel is within or near the working area | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 17/12/19 |  |
|  |  | Struck by load due to unsafe slinging | Bodily Injury & Damage to Property | - Entire crew is trained and have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent from dislodgement - To ensure load is rig at the centre of gravity | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 17/12/19 |  |
| 3 | Transportation of laser table to new location | Unstable load during moving or turning | Bodily Injury & Damage to Property when load topper | - Ensure laser table is secured firmly on the wooden base with support woodblock and secured with strapping belt to ensure it does not topper - Ensure laser table are well balanced on the wooden base before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point - Buddy system-one to control the pallet truck steering while the others to push and hold onto the table | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 17/12/19 |  |
|  |  | Passageway block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 17/12/19 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 17/12/19 |  |
| 4 | Lift down laser table and insert back air coating | Falling load | Bodily Injury & Damage to Property | - Known the CG of the load - To ensure granite table is secured firmly and balanced well on the lifter fork - Lift granite table at slow speed - To avoid sudden jerk of the load while being lifted - Ensure no unauthorised personnel is near or within the working area | 4 | 2 | 8 | Close supervision by supervisor to ensure all precaution are in place | 4 | 1 | 4 | Supervisor on site | 17/12/19 |  |