Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 5 Tukang Innovation Grove | Moving Operation | 1 | Reverse cometto to loading bay |  |
|  |  |  | 2 | Jack up engine using hydraulic jack |  |
|  |  |  | 3 | Transportation of engine from cometto to designated location |  |
|  |  |  | 4 | Using forklift to assist in pulling the engine |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 5 Tukang Innovation Grove Caterpillar Marine Asia Pacific Pte Ltd | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: Director |  |
| Next Review Date | 30/06/2023 |  | Date: 19/02/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Reverse cometto to loading bay | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - To clear all obstruction in way of moving path - Drive at slow speed when reversing cometto to loading bay - Ensure driver is guided by other staff | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the operation area | 4 | 1 | 4 | Supervisor on site | 22-23/ 02/21 |  |
| 2 | Jack up engine using hydraulic jack | Engine topple due to lost of balanced upon lifted up | Bodily Injury & Damage to Property | - Ensure jack use is in good working condition - Workers to be alert when jack up engine - Ensure engine are jack up at the right angle of gravity - Jack up engine to 4” high from one side to another is carried out in stages with the use of hard wooden block to insert under engine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 22-23/ 02/21 |  |
| 3 | Transportation of engine from cometto to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on engine base - Ensure rollers are properly secured to engine base with bolts and nuts to prevent from rolling out - Ensure engine load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 22-23/ 02/21 |  |
|  |  | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the engine - Supervisor to ensure there are adequate personnel to be deploy for the job | 3 | 3 | 9 | Use machine puller or forklift to assist in pulling the machine | 3 | 2 | 6 | Supervisor on site | 22-23/ 02/21 |  |
|  |  | Passage way block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 22-23/ 02/21 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the roller steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of engine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 22-23/ 02/21 |  |
| 4 | Using forklift to assist in pulling the engine | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Forklift operator to check whether anyone is nearby or behind the forklift before proceeding to drive - Ensure working area is clear of obstruction or unauthorised personnel - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Provision of spotter to guide forklift operator when required | 4 | 1 | 4 | Supervisor on site | 22-23/ 02/21 |  |