Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 50 Defu Lane 7 | Moving operation | 1 | Connect/disconnect wiring on scissor platform |  |
|  |  |  | 2 | Load machine onto part onto scissor platform |  |
|  |  |  | 3 | Shift scissor platform to door entrance |  |
|  |  |  | 4 | Forklift to fork up part from scissor platform and put aside |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member : Zailani |  |  |  |
| Location: | 50 Defu Lane 7 Heng Long Leather Co. Pte Ltd | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 23/11/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Connect / disconnect wiring on scissor platform | Exposed to electrical wire | Electrical shock | - Inspect electric cord periodically and keep in good condition - To disconnect and secure power sources not required - Lead extension cable must not trail across walkway must be routed if required to prevent trips | 4 | 2 | 8 | Supervisor to ensure no one temper with electrical wiring | 4 | 1 | 4 | Supervisor on site | 25/11/17 |  |
| 2 | Load machine part onto scissor platform | Falling load | Bodily Injury & Damage to Property | - Machine part to be shift slowly onto scissor platform - Ensure machine part are load at the centre of scissor platform - Secured machine part with strapping if required - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Supervisor to | 4 | 1 | 4 | Supervisor on site | 25/11/17 |  |
| 3 | Shift scissor platform to door entrance | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on scissor platform base - Ensure machine part is stable on scissor platform before being transfer to door entrance | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 25/11/17 |  |
|  |  | Passage way block by material or tooling | Trip & Fall | - Passage way and working area to be keep clear of material or tooling - Proper housekeeping to be maintained during work - Oil spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting is clear from material or tooling | 2 | 1 | 2 | Supervisor on site | 25/11/17 |  |
| 4 | Forklift to fork up part from scissor platform and put aside | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller | 4 | 1 | 4 | Supervisor on site | 25/11/17 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 1 | 3 | Supervisor on site | 25/11/17 |  |