Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 3 Cleantech Loop | Cutting of stud using hand grinder | 1 | Cutting of stud using hand grinder |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Cutting of stud | RA Member: Zailani |  |  |  |
| Location: | 3 Cleantech Loop, Cleantech 2A ARTC | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 19/12/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 8 | Cutting of stud using hand grinder | Unsuitable tool use | Bodily Injury | - Supervisor to ensure tools provided to worker are fit for the purpose - Hand grinder use must be fitted with its proper guard as supplied by the manufacturer - Ensure hand grinder handle is in good condition for worker to have a firm grip | 3 | 2 | 6 | Worn out or damage tool must not be use and disposed | 3 | 1 | 3 | Supervisor on site | 19/12/25 |  |
|  |  | Spark from cutting stud | Bodily Injury & Damage to Property | - To obtain HWP from the host prior to commencement of work - To remove any combustible materials away from the work area - Use of long sleeve shirt - PPE (glove, safety goggle) provided and shall be worn by personnel operating the hand grinder while cutting the stud - Fire blanket will be placed near the work area - Supervisor to ensure no unauthorised personnel is nearby and to stay clear of work area | 3 | 2 | 6 | Regular enforcement on safety rules and regulation on site | 3 | 1 | 3 | Supervisor on site | 19/12/25 |  |