Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 70 Joo Koon Circle | Dismantling of machine parts using hydraulic gantry crane | 1 | Hoist down gantry crane from low-bed using lorry loader |  |
|  |  |  | 2 | Fixing I-beam onto gantry crane structure |  |
|  |  |  | 3 | Working at height to tighten nuts and lock beam onto gantry crane |  |
|  |  |  | 4 | Using gantry crane to dismantle machine parts |  |
|  |  |  | 5 | Transport machine to loading area |  |
|  |  |  | 6 | Lifting operation |  |
|  |  |  | 7 | Signalling of load |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Dismantling of machine parts using hydraulic gantry crane | RA Member: Zailani |  |  |  |
| Location: | 70 Joo Koon Circle Nakano Metal Press Pte Ltd | RA Member: Daniel Wong |  |  |  |
|  |  | RA Member: Wong Yen Siong Site/Lifting Supervisor |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 01/02/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Hoist down gantry crane from low-bed chassis using lorry loader | Failing of lifting equipment | Bodily Injury & Damage to Property | - Trained and competent lorry loader operator appointed - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of load to be to be lifted - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While slewing the crane, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
| 2 | Fixing I-beam onto gantry crane structure | Load topple | Bodily Injury & Damage to Property | - To ensure I-beam is securely slug, properly balanced and trimmed to prevent load from dislodgment - To ensure I-beam is rig at it centre of gravity - To avoid sudden jerk of the load - Supervisor to ensure no unauthorised personnel is near or under the suspended load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Workers are to be briefed before commencement of work | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
| 3 | Working at height to tighten nuts and lock beam onto gantry crane | Falling from height | Bodily Injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder must be fully extended and stand on ground level - Buddy system/one to hold onto the ladder while the other is working on the ladder - Use of safety harness with short lanyard and anchored it to strong anchorage point at all time | 4 | 2 | 8 | Provision of boom/ scissor lift as working platform to work at height | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
| 4 | Using gantry crane to dismantle machine to parts | Falling of lifting equipment | Bodily Injury & Damage to Property | - Appointed and trained lifting supervisor, rigger and signalman to be on site - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - Ensure hoist connected to gantry crane control is in good condition and properly secured - Only experience and skill personnel are allowed to operate the gantry crane - Load to be lift up at slow speed - Never stand close or under the suspended load - Ensure operation area are clear of unauthorised personnel | 4 | 2 | 8 | Clear instruction is given as when to lift up or lower down the gantry crane | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting equipment are within limit, when in doubt, always use a higher capacity equipment | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Use of chain block to perform stable lifting when required | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal - Use glove | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Wong Yen Siong | 08-12 /02/23 |  |
| 5 | Transport machine to loading bay using rollers | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured to machine bed with bolt and nuts to prevent from rolling out - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Excessive weight load | Sprain back if force moving heavy load | - Use the correct capacity rollers to ensure wheel and bearing can sustained weight of load - Proper tool such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Travelling ground is well protected with metal plate in contact with rollers at all time | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in pulling /pushing the machine | 3 | 1 | 3 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent worker from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the roller steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Wong Yen Siong | 08-12 /02/23 |  |
| 6 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the suspended load - PPE (safety helmet, safety shoe) provided and put on - Ensure operation area are clear of unauthorised personnel | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and have relevant skill, certificate and experiences require to carry out the tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of the sling from the hook | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |
|  |  | Hand caught by or in between loose parts of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal - Glove to be provided and use | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Wong Yen Siong | 08-12 /02/23 |  |
| 7 | Signalling of load | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Only qualified signalman to execute the operation - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie whenever signalman is not visible to crane operator | 4 | 1 | 4 | Wong Yen Siong | 08-12 /02/23 |  |