Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | Blk 2022 Bukit Batok St 23 | Lorry loader operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Signalling of load to location |  |
|  |  |  | 5 | Adverse weather condition |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader Operation | RA Member: Zailani |  |  |  |
| Location: | Blk 2022 Bukit Batok St 23 #04-118 Jack & Lee Engineering | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date: | 30/06/2023 |  | Date: | 22/09/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - To clear all obstruction in way of moving path - Observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle to be moved into site with the aid of banks man guiding the driver to location | 4 | 1 | 4 | Supervisor on site | 07/10/22 |  |
| 2 | Setting up of crane on site | Crane topple due to outrigger not fully extended | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate/timber mats as base plate - Crane operator to be on site when extending or retracting outrigger (must be able to see all operation procedure) - Lifting supervisor to verify and ensure that outrigger is fully extended before any commencing work is done | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane | 4 | 1 | 4 | Lifting Supervisor on site | 07/10/22 |  |
|  |  | Lifting area not cordon off | Bodily Injury | - To demarcate off lifting area with safety cones, red/white tape to ensure no entry to unauthorised personnel - Execute operation during non-peak hours if possible - Total closure of parking/ hoisting area when required | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement of traffic and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 07/10/22 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Lifting Supervisor on site | 07/10/22 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lift | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting Supervisor on site | 07/10/22 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew, such as lifting supervisor, rigger and signalman is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting Supervisor on site | 07/10/22 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - To coordinate well with crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting Supervisor on site | 07/10/22 |  |
| 4 | Signalling of load to location | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Use of walkie-talkie to communicate whenever signalman is not visible to crane operator - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Close supervision by site supervisor | 4 | 1 | 4 | Lifting Supervisor on site | 07/10/22 |  |
| 5 | Adverse weather condition | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping/strike by lightning | - To stop work immediately if there is heavy rain and lightning in the area - To evacuate to nearest shelter if there is lightning present | 4 | 2 | 8 | Supervisor to ensure all machineries are turned off and guide workers to the nearest shelter | 4 | 1 | 4 | Supervisor on site | 07/10/22 |  |