Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 149 Gul Circle | Moving / Lifting & Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Lift up machine using hydraulic jack |  |
|  |  |  | 3 | Insert rollers to machine bed |  |
|  |  |  | 4 | Transport machine to loading bay using rollers |  |
|  |  |  | 5 | Setting up of crane on site |  |
|  |  |  | 6 | Lifting operation – to hoist up machine onto wooden base |  |
|  |  |  | 7 | Signalling of load |  |
|  |  |  | 8 | Using forklift to fork out accessories |  |
|  |  |  | 9 | Cutting of stud using hand grinder |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting & Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 149 Gul Circle FMC Technologies Singapore Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 10/09/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premised | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 18-20 /09/24 |  |
| 2 | Jack up machine using hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard woodblock to be insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 18-20 /09/24 |  |
| 3 | Insert rollers to machine bed | Sharp edges of machine | Cuts from sharp edges | - PPE (safety glove) provided and put on - Keep hand/finger clear of sharp edges - Insert rollers to below machine base with extra care - Use of suitable hand glove - To tighten bolt and nut onto rollers with adjustable spanner | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Pinch point | Injury to hand | - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view on where you’re placing your hand at - To standby woodblock under machine bed and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 18-20 /09/24 |  |
| 4 | Transport machine to loading bay using rollers | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured onto machine bed with bolts and nuts - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed Ensure floor is properly sweep and clear of any debris on the floor in case rollers got stuck by the debris Additional use of forklift, to secured sling onto machine for controlling the moving speed of machine when going down any slope | 4 | 1 | 4 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Excessive weight load | Sprain Back if force moving heavy load | - Use the correct capacity rollers for the move - Proper tools such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt, if needed, to prevent sprain back or back ache | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in puling/pushing the machine if required | 3 | 1 | 3 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 2 | 4 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 2 | 4 | Supervisor on site | 18-20 /09/24 |  |
| 5 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats as base plate - PPE (safety shoe, safety helmet, glove) provided and put on | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | 18-20 /09/24 |  |
| 6 | Lifting operation – to hoist up machine onto wooden base | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No lifting is to be carried out during bad weather | 4 | 1 | 4 | Lifting supervisor on site | 18-20 /09/24 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of lifting plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 18-20 /09/24 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 18-20 /09/24 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 18-20 /09/24 |  |
| 7 | Signalling of load | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Load to be lift up slowly and lifting supervisor to ensure load does not hit any obstacles or rooftop when being raise - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie whenever signalman is not visible to crane operator | 4 | 1 | 4 | Lifting supervisor on site | 18-20 /09/24 |  |
| 8 | Using forklift to fork out accessories | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Known the CG of the load - Ensure forklift fork if aligned directly to the CG of load - Ensure load is well balanced on the fork - Elevate the load slightly and fully tilt the mast backward | 4 | 2 | 8 | Use extension fork if required to give greater stability of the load | 4 | 1 | 4 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Forklift operator to check whether anyone is near or behind the forklift before proceeding to drive - Ensure working area is clear of any obstruction and unauthorised personnel - Workers to be alert and to guide forklift operator to ensure load do not hit side or top of container when being load up | 4 | 2 | 8 | Press horn if necessary to warn others that forklift is approaching | 4 | 1 | 4 | Supervisor on site | 18-20 /09/24 |  |
| 9 | Cutting of stud using hand grinder | Unsuitable tool use | Bodily Injury | - Supervisor to ensure tools provided to worker are fit for the purpose - Hand grinder use must be fitted with its proper guard as supplied by the manufacturer - Ensure hand grinder handle is in good condition for worker to have a firm grip | 3 | 2 | 6 | Worn out or damage tool must not be use and disposed | 3 | 1 | 3 | Supervisor on site | 18-20 /09/24 |  |
|  |  | Spark from cutting stud | Bodily Injury & Damage to Property | - To obtain HWP from the host prior to commencement of work - To remove any combustible materials away from the work area - Use of long sleeve shirt - PPE (glove, safety goggle) provided and shall be worn by personnel operating the hand grinder while cutting the stud - Fire blanket will be placed near the work area - Supervisor to ensure no unauthorised personnel is nearby and to stay clear of work area | 3 | 2 | 6 | Regular enforcement on safety rules and regulation on site | 3 | 1 | 3 | Supervisor on site | 18-20 /09/24 |  |