Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | Blk 4009 A.M.K Ave 10 | Moving / Lorry Crane & Forklift Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Lift up machine using hand jack |  |
|  |  |  | 3 | Transport machine to loading bay using rollers |  |
|  |  |  | 4 | Forklift operation |  |
|  |  |  | 5 | Lifting operation |  |
|  |  |  | 6 | Signalling of load |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lorry Crane & Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | Blk 4009 A.M.K Ave 10 #01-37, #01-40 Royal Insignia Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 27/06/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premised | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
| 2 | Jack up machine using hand jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard woodblock to be insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
| 3 | Transport machine to loading bay using rollers | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured onto machine bed with bolts and nuts - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Excessive weight load | Sprain Back if force moving heavy load | - Use the correct capacity rollers for the move - Proper tools such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt, if needed, to prevent sprain back or back ache | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 1 | 3 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 2 | 4 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
| 4 | Forklift operation | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Ensure forklift fork are aligned directly to the lifting point of the load - Known the CG of the load - Ensure load is well balanced on the fork - Elevate the load slightly and fully tilt the mast backward | 5 | 2 | 10 | Use extension fork if required to give greater stability of the load | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Forklift operator to check whether anyone is near or behind the forklift before proceeding to drive - Workers to be alert and assist to guide forklift operator of any blind spot when proceeding to load up machine onto lorry - Press horn if necessary to warn others that forklift is approaching | 5 | 2 | 10 | Assign traffic controller to control movement of vehicle and people within the vicinity | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
| 5 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No operation is to be carried out during bad weather | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of crane - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of lifting plan to all lifting personnel on site to ensure that lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury& Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |
| 6 | Signalling of load | View obstruction or load swing |  | - Only qualified signalman to execute the operation - Taglines should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Use of walkie-talkie whenever signalman is not visible to crane operator | 5 | 1 | 5 | Wong Yen Siong / Koh Ban San / Ibrahim / Zahari | 15/08/25 - 16/08/25 |  |