Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 24 Pioneer Crescent | Moving / Lifting Operation | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Insert pallet truck |  |
|  |  |  | 3 | Transportation of machine to loading bay |  |
|  |  |  | 4 | Setting of crane outriggers |  |
|  |  |  | 5 | Lifting operation |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting Operation | RA Member : Zailani |  |  |  |
| Location: | 24 Pioneer Crescent #04-10 Ecopol Pte Ltd | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 15/02/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling Good coordination between workers | 4 | 1 | 4 | Supervisor on site | 12/04/17 |  |
| 2 | Insert pallet truck | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert pallet truck to below machine base with care - Use of suitable hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 12/04/17 |  |
| 3 | Transportation of machine to loading bay | Unstable load during moving / turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and is stable on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 12/04/17 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 2 | 4 | Supervisor on site | 12/04/17 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before it touches any obstacles | 2 | 2 | 4 | Supervisor on site | 12/04/17 |  |
| 4 | Setting up of crane | Hit by moving parts of crane | Bodily Injury | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats - PPE (etc.. safety shoe, safety helmet, glove) provided and put on | 3 | 2 | 6 | Ensure no unauthorised personnel near moving parts of crane | 3 | 1 | 3 | Lifting supervisor on site | 12/04/17 |  |
| 5 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 12/04/17 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 12/04/17 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at it centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 12/04/17 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting supervisor on site | 12/04/17 |  |
|  |  | View obstruction or poor visibility, load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 12/04/17 |  |