Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 18 Bedok South Rd | Moving & Lifting Operation | 1 | Using ‘A’ frame to dismantle tombstone |  |
|  |  |  | 2 | Jack up machine using hydraulic jack |  |
|  |  |  | 3 | Insert rollers to machine bed |  |
|  |  |  | 4 | Transportation of machine to loading area |  |
|  |  |  | 5 | Using forklift to assist in pulling/pushing machine |  |
|  |  |  | 6 | Setting of crane outriggers |  |
|  |  |  | 7 | Lifting operation |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving & Lifting Operation | RA Member: Zailani |  |  |  |
| Location: | No 18 Bedok South Rd Hamilton Sundstrand Pacific Aerospace Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 10/09/2018 |  | Designation: | Director |  |
| Next Review Date | 09/09/2019 |  | Date: | 10/09/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using ‘A’ frame to dismantle tombstone | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system when handling heavy equipment | 3 | 2 | 6 | Use of mechanical aid such as lifter if required | 3 | 1 | 3 | Wong Yen Siong | TBC |  |
|  |  | Working at height to secured sling onto tombstone | Fall from Height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder use is in good working condition - Ladder use must be fully extended and stand on level ground - Ensure no standing on top 2 rung of ladder - Buddy system-one to hold the ladder while the other is working on top of ladder - Use of safety helmet while working at height | 4 | 2 | 8 | Close supervision by supervisor and to ensure all safety precaution are in place | 4 | 1 | 4 | Wong Yen Siong | TBC |  |
|  |  | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - To ensure lifting equipment use are in compliance with weight of tombstone - Ensure no unauthorised personnel is near the working area | 4 | 2 | 8 | When lifting of tombstone, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong | TBC |  |