Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 30 & 32 Tuas Ave 8 | Moving / Forklift Operation | 1 | Lift up Machine Using Crowbar |  |
|  |  |  | 2 | Transportation of machine to loading bay |  |
|  |  |  | 3 | Using forklift to fork up for packing and load onto Malaysia lorry |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 30 & 32 Tuas ave 8 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 16/03/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine/ Equipment using crowbar | Load topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 16/03/20 |  |
| 2 | Transportation of machine to loading area | Unstable load during moving or turning | Bodily injury & Damage to property if machine topper | - Ensure pallet truck are place accordingly on machine base -Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to loading bay -Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 16/03/20 |  |
| 3 | Using forklift to fork up and load onto Malaysia lorry | Passageway block by equip[ment and tool | Bodily Injury & Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job -Proper housekeeping to be maintained during work -oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transportation before moving and double check clearing of all materials and tooling. | 2 | 1 | 2 | Supervisor on site | 16/03/20 |  |
|  |  | Obstruction to vision while moving | Damage to property | - To keep close lookout on route of travel -Ensure movement is guided -To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 16/03/20 |  |
|  |  | Falling load | Bodily injury & damage to property | -Only certified and competent forklift driver are allowed to operate the forklift -Known the CG of the load -Elevate the load slightly and full tilt the mast backward -Ensure load weight are spread evenly and balanced well on the fork -Ensure no unauthorised personnel is near or within the working area | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 16/03/20 |  |