Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 18 Teban Gardens Crescent | Moving / lifting operation | 1 | Insert pallet truck |  |
|  |  |  | 2 | Transportation of machine to loading bay |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Using battery lifter to lift up table and insert pad |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / lifting operation | RA Member: Zailani |  |  |  |
| Location: | 18 Teban Gardens Crescent Setsco Services Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 21/04/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Insert pallet truck onto machine stand | Sharp edges | Minor cut or bruises to hand | - Keep hand/finger clear of sharp edges - Insert pallet truck to below machine base with care - Use of suitable hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Zahari (Supervisor on site) | TBA |  |
| 2 | Transportation of machine to loading bay | Unstable load during moving / turning | Bodily injury if machine topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Buddy system-one to control the pallet truck steering while the others to push and hold onto the machine - Workers to be alert and keep close lookout on route of travel from point to point | 5 | 2 | 10 | Slow moving speed | 5 | 1 | 5 | Zahari (Supervisor on site) | TBA |  |
|  |  | Trip & fall | Sprain & strain | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Any oil or water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Zahari (Supervisor on site) | TBA |  |
|  |  | Obstruction to vision while moving | Damage to property if hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Zahari (Supervisor on site) | TBA |  |
| 3 | Lifting operation | Failing of lifting equipment | Damage to property | - All lifting equipment use are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Rigger to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load | 5 | 2 | 10 | While slewing the crane, try not to jerk the load | 5 | 1 | 5 | Zahari (Lifting Supervisor on site) | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Damage to property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Rigger to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 5 | 1 | 5 | Zahari (Lifting supervisor on site) | TBA |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Cut or bruises to hand | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose part of lifting gears | 3 | 1 | 3 | Zahari (Lifting supervisor on site) | TBA |  |
|  |  | Load swing | Damage to property | - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 5 | 1 | 5 | Zahari (Lifting supervisor on site) | TBA |  |
| 4 | Using battery lifter to lift up table and insert pad | Falling load | Damage to property | - Insert lifter fork to below table with care to prevent fork hitting granite table - Known the CG of the load - Supervisor to ensure granite table is well balance on the fork - To lift up table at slow speed - To avoid sudden jerk of the load while being lifted up or down - Ensure no unauthorised personnel is near or within the working area | 5 | 2 | 10 | Close supervision by supervisor to ensure all safety precaution are in place | 5 | 1 | 5 | Zahari (Supervisor on site) | TBA |  |