Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 21 Tanjung Penjuru | Moving / Lifting Operation | 1 | Jack up machine using hydraulic jack |  |
|  |  |  | 2 | Insert rollers to machine bed |  |
|  |  |  | 3 | Transport machine to loading bay using rollers |  |
|  |  |  | 4 | Using forklift to assist in pulling/pushing machine |  |
|  |  |  | 5 | Setting up of crane on site |  |
|  |  |  | 6 | Lifting operation |  |
|  |  |  | 7 | Signalling of load |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting Operation | RA Member: Zailani |  |  |  |
| Location: | 21 Tanjung Penjuru Singapore Highpolymer Chemical Products | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 26/04/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine using hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard wooden block to insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 28-30 /04/21 |  |
| 2 | Insert rollers to machine bed | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert rollers to below machine base with care - Use cotton glove - Remove woodblock only when rollers are properly secured onto machine bed | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 28-30 /04/21 |  |
| 3 | Transport machine to loading bay using rollers | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured to machine bed with bolt and nut to prevent from rolling out - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 28-30 /04/21 |  |
|  |  | Excessive weight load | Sprain back if force moving heavy load | - Use the correct capacity rollers to ensure rollers wheel and bearing can sustained weight of load - Proper tool such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Ensure travelling ground is well protected with metal plate in contact with rollers at all time | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 1 | 3 | Supervisor on site | 28-30 /04/21 |  |
|  |  | Trip & fall | Sprains & fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent worker from tripping | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 3 | 1 | 3 | Supervisor on site | 28-30 /04/21 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 28-30 /04/21 |  |
| 4 | Using forklift to assist in pulling/ pushing machine | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Forklift operator to check whether anyone is near or behind the forklift before proceeding to drive - Ensure working area is clear of any obstruction and unauthorised personnel - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Provision of spotter to guide the forklift operator when required | 4 | 1 | 4 | Supervisor on site | 28-30 /04/21 |  |
| 5 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams to be fully extended and resting on steel plate/timber mats as base plate - PPE (safety shoe, safety helmet, glove) provided and put on | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | 28-30 /04/21 |  |
| 6 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 28-30 /04/21 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 28-30 /04/21 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of the sling from the hook | 4 | 1 | 4 | Lifting supervisor on site | 28-30 /04/21 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use cotton glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 28-30 /04/21 |  |
| 7 | Signalling of load | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline for load when necessary to prevent of load swing - Clear all obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Use of walkie-talkie to communicate whenever signalman is not visible to crane operator | 4 | 2 | 8 | Close supervision by lifting supervisor | 4 | 1 | 4 | Lifting supervisor on site | 28-30 /04/21 |  |