Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 10 Dover Rd – 1 Punggol Coast Rd | Moving Operation | 1 | Transport of lorry into or out of site premise |  |
|  |  |  | 2 | Lift up equipment using crowbar |  |
|  |  |  | 3 | Transport equipment to loading area |  |
|  |  |  | 4 | Load up equipment onto tailgate lorry |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 10 Dover Rd, Singapore Institute of Technology 1 Punggol Coast Rd, Singapore Institute of Technology | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 23/01/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lorry into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | -To coordinate with occupier for safe route access - To clear all obstruction in way of moving path - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 17/03/25 |  |
| 2 | Lift up equipment using crowbar | Load topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up equipment - Ensure equipment are lift up at the right angle of gravity - Lift up equipment to 3” high from one side to another is carried out in stages with the use of hard wooden block - PPE (safety shoes, glove) provided and put on | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 17/03/25 |  |
| 3 | Transport equipment to loading area | Unstable load during moving or turning | Bodily Injury & Damage to Property if equipment topper | - Ensure pallet truck are place accordingly on equipment base - Ensure equipment load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 17/03/25 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 17/03/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 17/03/25 |  |
| 4 | Load up equipment onto tailgate lorry | Mechanical failure | Bodily Injury & Damage to Property | - To ensure that any tail lift is suitable for the task to be taken and has the capacity to carry the required loads - SWL of tailgate are display on the platform - Driver are to strictly follow the manufacture and operation instructions | 4 | 2 | 8 | In case the tail lift is not functioning properly, to stop at once and informed supervisor immediately | 4 | 1 | 4 | Supervisor on site | 17/03/25 |  |
|  |  | Struck by load | Bodily Injury & Damage to Property | - Workers are trained and to have relevant skill and experiences required to carry out tasks - When moving the load onto platform or unloading at the ground, push rather than pull the load to avoid the load rolling or falling over - The load on the platform is positioned an equal distance from each side and as close as possible to the vehicle - Only one item is moved at a time onto the tailgate platform - Unauthorised personnel to stay well clear of operation area - PPE (safety shoes, glove) provided and put on | 4 | 2 | 8 | Close supervision and clear communication is given by supervisor to worker | 4 | 1 | 4 | Supervisor on site | 17/03/25 |  |