Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 70 Tuas South St 5 | Moving Operation | 1 | Lift up furnace using crowbar |  |
|  |  |  | 2 | Transportation of furnace to loading bay |  |
|  |  |  | 3 | Lifting operation to load furnace onto wooden base |  |
|  |  |  | 4 | Drilling to secured furnace onto base |  |
|  |  |  | 5 | Use of nail gun |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 70 Tuas South St 5 Tecnisco Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 26/02/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up furnace using crowbar | Topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up furnace - Ensure furnace are lift up at the right angle of gravity - Lift up furnace to 3” high from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 28/02/19 |  |
| 2 | Transportation of furnace to loading area | Unstable load during moving / turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on furnace base - Ensure load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 28/02/19 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 28/02/19 |  |
| 3 | Lifting operation to load furnace onto wooden base | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of load - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | 28/02/19 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Supervisor on site | 28/02/19 |  |
| 4 | Drilling to secured furnace onto base | Continuous noise generated | Noise induce hearing lost | - PPE (etc.. ear plug) provided | 3 | 2 | 6 | Rotating shift of workers | 3 | 1 | 3 | Supervisor on site | 28/02/19 |  |
|  |  | Faulty parts of pneumatic tool | Bodily Injury due to tool accidentally disconnected | - Ensure hose connection for pneumatic tools are fastened securely to pipe outlet | 3 | 2 | 6 | Regular inspection of hose connection | 3 | 1 | 3 | Supervisor on site | 28/02/19 |  |
| 5 | Use of nail gun | Mishandling of nail gun | Bodily Injury to workers or nearby personnel | - Only trained and competent personnel are allowed to use tools that are issued to them - Understudy personnel need the supervision of a trained personnel - To ensure possible routes of projectiles are clear of personnel in case of miss fire | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel are near the working area | 4 | 1 | 4 | Supervisor on site | 28/02/19 |  |