Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 16 Seletar Aerospace Crescent | Moving / Lifting Operation | 1 | Lift up machine using hydraulic jack |  |
|  |  |  | 2 | Insert rollers |  |
|  |  |  | 3 | Transportation of machine to loading bay |  |
|  |  |  | 4 | Setting of crane outriggers |  |
|  |  |  | 5 | Lifting operation |  |
|  |  |  | 6 | Load machine onto container |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting Operation | RA Member: Zailani |  |  |  |
| Location: | 16 Seletar Aerospace Crescent JEP Precision Engineering Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/06/2020 |  | Designation: | Director |  |
| Next Review Date | 30/05/2023 |  | Date: | 21/07/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine using hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard wooden block to insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 28-29 07/20 |  |
| 2 | Insert rollers | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert rollers to below machine base with care - Use of suitable hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 28-29 07/20 |  |
| 3 | Transportation of machine to loading bay | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are properly secured to machine base with bolt and nut to prevent from rolling out - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 28-29 07/20 |  |
|  |  | Excessive weight load | Sprain Back when pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate personnel to be deploy for the job | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 1 | 3 | Supervisor on site | 28-29 07/20 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 28-29 07/20 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 28-29 07/20 |  |
| 4 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams to be fully extended and resting on steel plate/timber mats - Lifting zone to be barricade, only relevant personnel is allowed to enter the lifting zone - PPE (safety shoe, safety helmet, glove) provided and put on | 3 | 2 | 6 | Supervisor to ensure no unauthorised personnel or vehicle near moving parts of crane | 3 | 1 | 3 | Lifting supervisor on site | 28-29 07/20 |  |
| 5 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 28-29 07/20 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 28-29 07/20 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine are rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 28-29 07/20 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 28-29 07/20 |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 28-29 07/20 |  |
| 6 | Load machine onto container | Machine hitting side of container | Damage to Property | - Ensure movement is guided - Workers to be alert and keep close lookout to ensure machine is moving in a straight direction | 3 | 2 | 6 | Slow moving speed | 3 | 1 | 3 | Lifting supervisor on site | 28-29 07/20 |  |
|  |  | Using crane to assist in pushing machine | Bodily Injury & Damage to Property | - Supervisor to ensure crane operator to operate crane at slow speed - To ensure no workers or unauthorised personnel is near the crane incase crane swing - Ensure signalman give proper means of signal to crane operator | 4 | 2 | 8 | Close supervision and regular reminder by supervisor to ensure all procedure are in place | 4 | 1 | 4 | Lifting supervisor on site | 28-29 07/20 |  |