Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 285 Jalan Ahmad Ibrahim | Moving & Forklift Operation | 1 | Lift up machine using hydraulic jack |  |
|  |  |  | 2 | Insert rollers to machine bed |  |
|  |  |  | 3 | Transport machine to loading bay |  |
|  |  |  | 4 | Forklift operation – to fork up machine and transfer to carpark area |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving & Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 285 Jalan Ahmad Ibrahim Panasonic Factory Solutions Asia Pacific | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 04/03/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using hydraulic/ hand jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of hard wooden block to insert under machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |
| 2 | Insert rollers to machine bed | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert rollers to below machine base with care - Use of suitable and glove - Remove wooden block only when rollers are properly secured onto machine bed | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |
| 3 | Transport machine to loading bay | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topple | - Ensure rollers are place accordingly on machine bed - Ensure machine load are spread evenly and balanced well on the rollers before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Excessive weight load | Sprain back if force moving heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate personnel to be deploy for the job - Proper tool such as manual puller to be brought to site - Use of waist belt if needed to prevent sprain back or backache | 3 | 2 | 6 | Use of manual puller when required to assist in pulling the machine | 3 | 1 | 3 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Trip & Fall | Sprains & strains | - Passageway and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - If possible keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 3 | 2 | 6 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling within the work area | 3 | 1 | 3 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Laying of canvas / metal sheet onto flooring | Cuts from sharp edges | - To replace defective sheet with a new one - PPE (glove) to be provided and put on when carrying metal sheet - Buddy system-two worker to lift one piece of metal sheet and lay down onto flooring at a time | 2 | 2 | 4 | Regular check by storeman and ensure equipment issue is in good condition | 2 | 1 | 2 | Supervisor on site | 20/03/24 - 27/03/24 |  |
| 3 | Forklift operation – to fork up machine and transfer to new location | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Worker to guide forklift driver to ensure that the fork is properly adjust and aligned to the lifting point of the machine - To ensure machine load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork when needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure no unauthorised personnel is within the working zone - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift if approaching | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other stuff - If required, reverse driving with reverse light on - Drive forklift at slow moving speed | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 20/03/24 - 27/03/24 |  |