Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 39 Changi North Crescent | Moving / Forklift Operation | 1 | Transport out surface table to loading bay using pallet truck |  |
|  |  |  | 2 | Forklift operation – to load up surface table onto lorry chassis |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 39 Changi North Crescent Goodrich Aerospace | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 02/11/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport out surface table to loading bay using pallet truck | Unstable load during moving or turning | Bodily Injury & Damage to Property if load topper | - Ensure pallet truck are place accordingly on surface table bed - Ensure surface table load are spread evenly and balanced well on the pallet truck before being transport to loading area - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 09/11/23 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Any oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 09/11/23 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 09/11/23 |  |
| 2 | Forklift operation – to load up surface table onto lorry chassis | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure surface table load are spread evenly and balanced well on the fork - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 09/11/23 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller when required, to control movement of vehicle and people within the vicinity | 4 | 1 | 4 | Supervisor on site | 09/11/23 |  |