Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 10 Science Pk Rd | Moving / Lifting Operation | 1 | Transport case to loading area |  |
|  |  |  | 2 | Lifting operation – to hoist up case onto lorry chassis |  |
|  |  |  | 3 | Using customer overhead crane to hoist up case to 3rd floor |  |
|  |  |  | 4 | Unload machine from base |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting Operation | RA Member : Zailani |  |  |  |
| Location: | 10 Science Park Rd #02-06 - 38F Tuas Ave 11 | RA Member : Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 14/04/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport case to loading area | Unstable load during moving or turning | Bodily Injury & Damage to Property if load topple | - Ensure pallet truck are place accordingly on the case - Ensure case load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 14/05/22 |  |
|  |  | Passage way block by material and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 14/05/22 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of equipment so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 14/05/22 |  |
| 2 | Lifting operation – to hoist up case onto lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load to be lifted - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 14/05/22 |  |
|  |  | Struck by objects due to unsafe rigging | Bodily Injury & Damage to Property | - Entire lifting crew is such as rigger, signalman and lifting supervisor are trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load are rig at the centre of gravity - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 14/05/22 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 14/05/22 |  |
| 3 | Using customer overhead crane to hoist up case to 3rd floor | Overloading | Bodily Injury & Damage to Property | - To ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 14/05/22 |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path of overhead crane | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method | 4 | 1 | 4 | Lifting supervisor on site | 14/05/22 |  |
| 4 | Unload machine from base | Falling load | Damage to Property | - Use of ramp to roll down machine from base - Ensure ramp are properly secure to the base - Machine to be push down slowly - To ensure there are adequate personnel to be deploy for the job | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel is within the working area | 4 | 1 | 4 | Supervisor on site | 14/05/22 |  |