Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 9 Woodlands Ave 9 | Moving / lifting operation | 1 | Insert pallet truck to machine base |  |
|  |  |  | 2 | Transport machine to loading bay |  |
|  |  |  | 3 | Lifting operation – to hoist up/down machine from lorry chassis |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Lifting Operation | RA Member : Zailani |  |  |  |
| Location: | 9 Woodlands Ave 9 Republic Polytechnic | RA Member : Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 14/12/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Insert pallet truck to machine base | Sharp edges | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert pallet truck to below equipment base with care - Use of cotton hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 16/12/21 |  |
| 2 | Transport machine to loading bay | Unstable load during moving or turning | Bodily Injury & Damage to Property if equipment topple | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 16/12/21 |  |
|  |  | Passage way block by material and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 16/12/21 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of equipment so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 16/12/21 |  |
| 3 | Lifting operation – to hoist up/down machine onto lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load to be lifted - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 16/12/21 |  |
|  |  | Struck by objects due to unsafe rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load are rig at the centre of gravity - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 16/12/21 |  |
|  |  | Hand caught by or in between loose part of sling | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 16/12/21 |  |