Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 18 Ayer Rajah Crescent | Moving / Forklift Operation | 1 | Shift out X-ray machine to loading bay |  |
|  |  |  | 2 | Forklift operation - to fork X-ray machine from loading bay and load onto base |  |
|  |  |  | 3 | Packing of case |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manger | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 18 Ayer Rajah Crescent Pepperl + Fuchs Asia Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/06/2020 |  | Designation: | Director |  |
| Next Review Date | 30/05/2023 |  | Date: | 17/08/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Shift out X-ray machine to loading bay | Unstable load while moving / turning | Bodily injury & damage to property | - Ensure pallet truck are place accordingly on machine base - Ensure load is stable and balanced well on the pallet truck before being transport to loading bay - Buddy system-one to control the pallet truck steering while the others to push the load - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 20/08/20 |  |
|  |  | Passage way block by material and tools | Trip & fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 20/08/20 |  |
| 2 | Forklift operation - to fork up X-ray machine from loading bay and load onto base | Falling load | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - To ensure machine is well balanced on the fork - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 20/08/20 |  |
|  |  | Hitting people or property | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel | 4 | 2 | 8 | Assignment of traffic controller when required to guide and control movement within the vicinity | 4 | 1 | 4 | Supervisor on site | 20/08/20 |  |
| 3 | Packing to form case | Continuous noise generated when drilling | Noise induce hearing lost | - PPE (ear plug) provided and put on | 3 | 2 | 6 | Rotating shift of workers | 3 | 1 | 3 | Supervisor on site | 20/08/20 |  |
|  |  | Mishandling of nail gun | Bodily injury to worker or nearby personnel | - Only trained and competent person are allowed to use tools that are issued to them - Understudy personnel need the supervision of a trained personnel - To ensure possible routes of projectiles are clear of unauthorised personnel in case of miss fired | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel are near or within the working area | 4 | 1 | 4 | Supervisor on site | 20/08/20 |  |
|  |  | Working on ladder | Bodily Injury if fall from ladder | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on ground level - Buddy system-one to hold onto the ladder while the other is working on the ladder | 4 | 2 | 8 | Close supervision and regular reminder by supervisor | 4 | 1 | 4 | Supervisor on site | 20/08/20 |  |
|  |  | Hot vacuum sealer sealing the vacuum bag | Burnt | - Follow proper procedure of vacuum sealer - Only trained and competent personnel are allowed to use tools that are issued to them | 3 | 2 | 6 | Close supervision by supervisor | 3 | 1 | 3 | Supervisor on site | 20/08/20 |  |