Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 7 Sunview Rd | Relocate equipment within factory premise | 1 | Lift up equipment using crowbar/hand jack |  |
|  |  |  | 2 | Insert rollers/pallet truck to equipment bed |  |
|  |  |  | 3 | Laying of canvas/pvc sheet onto flooring |  |
|  |  |  | 4 | Relocate equipment within factory premise |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocate equipment within factory premise | RA Member: Zailani |  |  |  |
| Location: | 7 Sunview Rd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 26/03/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up equipment using crowbar/hand jack | Load topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up load - Ensure load are jack up at the right angle of gravity - Jack up load to 4” high from one side to another is carried out in stages with the use of hard wooden block to insert under tumbler bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 02/04/25 |  |
| 2 | Insert rollers/ pallet truck to equipment bed | Sharp edges of tumbler | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert rollers to below tumbler bed with care - Use of suitable hand glove - Remove woodblock only when rollers are properly secured onto tumbler bed | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 02/04/25 |  |
| 3 | Laying of canvas/pvc sheet onto flooring | Sharp edges | Cuts | - To replace defective sheet with a new one - Deploy additional manpower to assist on laying pvc sheet to minimize weight carry on each individual - Buddy system-two worker to lift one pcs of pvc sheet and lay out onto flooring one at a time - Wearing of glove | 2 | 2 | 4 | Regular check by storeman and ensure equipment issue is in good condition | 2 | 1 | 2 |  | 02/04/25 |  |
| 4 | Relocate equipment within factory premise | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on equipment bed - Ensure rollers are properly secured onto equipment bed with bolts and nuts - Ensure equipment load are spread evenly and balanced well on the rollers before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 02/04/25 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 02/04/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 02/04/25 |  |