Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 80 Tuas West Dr | Hoisting/unloading of machine using factory 10ton overhead crane | 1 | Container reversing into inside factory premise |  |
|  |  |  | 2 | Open up container canvas |  |
|  |  |  | 3 | Working on top container to secured sling |  |
|  |  |  | 4 | Use of factory 10ton overhead crane to hoist and unload machine and parts |  |
|  |  |  | 5 | Signalling of load to drop off point |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Hoisting/unloading of machine using factory 20ton overhead crane | RA Member: Zailani |  |  |  |
| Location: | 80 Tuas West Dr Innovex Asia Pacific Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 01/10/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  |  | Risk Control |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls |  | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Container reversing into inside factory premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to site Property | - To coordinate with the occupier for safe route access - To clear all obstruction in way of vehicle moving path - Reverse container at slow speed when reversing into factory premise - To ensure height clearance of door entrance is high enough for container to move in and out of factory | 4 | 2 | 8 | Provision of banks man to guide the driver to location |  | 4 | 1 | 4 | Supervisor on site | Estimate: 08-10 /10/25 |  |
| 2 | Open up container canvas | Fall from height | Bodily Injury | - Provision of scissor lift as working platform to work at height - Only certified and competent scissor lift operator are allowed to operate the scissor lift - Worker working at height on top scissor lift to put on proper PPE (safety harness, safety helmet) | 5 | 2 | 10 | All trained and authorised operator to be paste on the scissor lift |  | 5 | 1 | 5 | Supervisor on site | Estimate: 08-10 /10/25 |  |
| 3 | Working on top container to secured sling | Fall from height | Bodily Injury | - Worker working at height on top container to secured sling onto machine to put on proper PPE (safety harness, safety helmet) - Ensure safety harness use are properly strap on - Ensure safety harness are properly secured onto lashing rings at top of the container to act as anchorage point | 5 | 2 | 10 | Full supervision and regular reminder by supervisor to ensure all safety precaution are to followed and in place |  | 5 | 1 | 5 | Supervisor on site | Estimate: 08-10 /10/25 |  |
| 4 | Use of factory 20ton overhead crane to hoist and unload machine | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, do it smoothly without sudden acceleration or deceleration Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation |  | 5 | 1 | 5 | Lifting supervisor on site | Estimate: 08-10 /10/25 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 5 | 2 | 10 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane |  | 5 | 1 | 5 | Lifting supervisor on site | Estimate: 08-10 /10/25 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong onsite personnel only |  | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 08-10 /10/25 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to overhead crane operator - To coordinate well with the crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose part of sling |  | 3 | 1 | 3 | Lifting supervisor on site | Estimate: 08-10 /10/25 |  |
| 5 | Signalling of load to drop off point | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Load to be lift up slowly - Tagline shall be install at both side of load when required to gain control while being hoist up - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method | 5 | 2 | 10 | Use of walkie-talkie to communicate whenever signalman is not visible to overhead crane operator |  | 5 | 1 | 5 | Lifting supervisor on site | Estimate: 08-10 /10/25 |  |