Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 12 Ayer Rajah Crescent | Forklift operation | 1 | Unloading case from lorry chassis to the ground |  |
|  |  |  | 2 | Uncrate case |  |
|  |  |  | 3 | Transport machine into factory premise |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 12 Ayer Rajah Crescent Thales Singapore | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 12/05/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Unloading case from lorry chassis to the ground | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure load weight are spread evenly and balanced well on the fork - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 15/05/23 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller when required | 4 | 1 | 4 | Supervisor on site | 15/05/23 |  |
| 2 | Uncrate case | Falling of wooden plank | Bodily Injury | - PPE (safety helmet, safety shoe) provided and put on - Uncrate and dismantle case in correct sequence - Remove immediately any loose plank | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 15/05/23 |  |
|  |  | Protruding nail on loose timber | Bodily Injury | - PPE (safety shoe) provided and put on - Remove immediately any loose timber with protruding nails from working area - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Close supervision and regular reminder by supervisor | 3 | 1 | 3 | Supervisor on site | 15/05/23 |  |
| 3 | Transport machine into factory premise | Unstable load during moving or turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on the machine base - Ensure machine load are spread evenly and balance well on the pallet truck before being transport to location - Buddy system-one to control the pallet truck steering while the other to push and hold onto the machine | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 15/05/23 |  |
|  |  | Passage way block by materials or tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before work commence - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Ensure travelling ground is protected with PVC plate in contact with pallet truck rollers at all time to prevent damage on tiles flooring | 2 | 2 | 4 | Supervisor to make sure route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 15/05/23 |  |
|  |  | Obstruction to vision while moving | Damage to Property if hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 15/05/23 |  |