Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 100 Tuas South Ave 8 | Dismantling of machine parts using factory 40ton overhead crane | 1 | Working at height |  |
|  |  |  | 2 | Use of factory overhead crane to hoist machine parts |  |
|  |  |  | 3 | Signalling of load to drop off point |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project manager | Approved by: Eric Wong Signature: |  | Reference Number: EW/Q/0573/ 20 |
| --- | --- | --- | --- | --- | --- |
| Process: | Dismantling of machine parts using factory 40 ton overhead crane | RA Member: Zailani |  |  |  |
| Location: | 100 Tuas South Ave 8 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 01/09/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Working at height | Fall from height | Bodily Injury | - PTW to be obtained and approved by Host Authorised personnel before commencement of work - Provision of boom lift as working platform to work at height - Only certified and competent boom lift operator are allowed to operate the boom lift - Worker working at height are trained and must have WAH course - Workers working at height on top boom lift to put on proper PPE (safety harness, safety helmet) - Ensure safety harness are properly strapped on and the lanyard is fully secured to the bucket at all time while working at height | 4 | 2 | 8 | All trained and authorised operator record to be paste on the boom lift | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
| 2 | Use of factory overhead crane to hoist machine parts | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Lifting gears use such as sling to be protected from sharp edges of load | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong onsite personnel only | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure proper means of signal are given to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose part lifting gears | 3 | 1 | 3 | Wong Yen Siong / Koh Ban San | TBA |  |
| 3 | Signalling of load to drop off point | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Only qualified signalman to execute the operation | 4 | 2 | 4 | Close supervision by supervisor | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline shall be installed at both side of load to gain control while being elevated and or while at height - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 4 | While slewing the crane, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |