Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 22 Jalan Terusan | Hoisting/unloading of machine using factory 38ton overhead crane | 1 | Container reversing into inside factory premise |  |
|  |  |  | 2 | Use of factory 38ton overhead crane to hoist and unload machine and parts |  |
|  |  |  | 3 | Signalling of load to drop off point |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Hoisting/unloading of machine using factory 38 ton overhead crane | RA Member: Zailani |  |  |  |
| Location: | 22 Jalan Terusan NOV | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 10/08/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Container reversing into inside factory premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to site Property | - To clear all obstruction in way moving path - Reverse container at slow speed when reversing into factory premise - To ensure height clearance of door entrance is height enough for container to move in and out of factory | 3 | 2 | 6 | Provision of banks man to guide the driver | 3 | 1 | 3 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |
| 2 | Use of factory 38ton overhead crane to hoist and unload machine and parts | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong onsite personnel only | 4 | 1 | 4 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure proper means of signal are given to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose part lifting gears | 3 | 1 | 3 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |
| 3 | Signalling of load to drop off point | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Only qualified signalman to execute the operation - Tagline shall be install at both side of load when required to gain control while being hoist up - Clear obstruction and unauthorised personnel in way of lifting path | 4 | 2 | 4 | Close supervision by supervisor | 4 | 1 | 4 | Wong Yen Siong / Zahari / Koh Ban San | 17-20 /08/21 |  |