Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 32 Tuas Ave 4 | Overhead crane operation | 1 | Use of factory overhead crane to hoist out chip conveyor |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Overhead crane operation | RA Member: Zailani |  |  |  |
| Location: | 32 Tuas Ave 4 NTS Amega West Singapore Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 12/06/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
|  | Use of factory overhead crane to hoist out conveyor | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of load to be lifted - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | 15-16 /06/23 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Supervisor on site | 15-16 /06/23 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire crew is trained and to have relevant skill and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal by Eric Wong personnel only | 4 | 1 | 4 | Supervisor on site | 15-16 /06/23 |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to overhead crane operator through hand signalling method | 4 | 1 | 4 | Supervisor on site | 15-16 /06/23 |  |