Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 133 Tuas South Ave 3 | Lorry Loader / Forklift Operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Lifting operation – to unload forklift from lorry chassis |  |
|  |  |  | 3 | Forklift operation - to fork down vacuum pump from lorry and up slope |  |
|  |  |  | 4 | Transport vacuum pump to pump room by using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manger | Approved by: Eric Wong Signature: |  | Reference Number: EW/Q/0636/ 21 |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 133 Tuas South Ave Ciba Vision Asian Manufacturing & Logistics Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/06/2020 |  | Designation: | Director |  |
| Next Review Date | 30/05/2023 |  | Date: | 16/11/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily injury & damage to property | - Vehicle to be moved into site with the aid of banks man guiding the driver when required - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 25/11/21 |  |
| 2 | Lifting operation – to unload forklift from lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - No defective gears allowed - Riggers to ensure lifting gears use are in compliance with weight of load - Never stand close or under the load - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 25/11/21 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - Lifting plan is pre-briefed to all lifting personnel on side before start work - PTW to be obtained and approved by Host Authorised personnel before commencement of work | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 25/11/21 |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 25/11/21 |  |
| 3 | Forklift operation - to fork down vacuum pump from lorry and ramp up slope | Falling load | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - To ensure vacuum pump is well balanced on the fork - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load when going up slope | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 25/11/21 |  |
|  |  | Hitting people or property | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement within the vicinity when required | 4 | 1 | 4 | Supervisor on site | 25/11/21 |  |
| 4 | Transport vacuum pump to pump room by using pallet truck | Unstable load while moving / turning | Bodily injury & damage to property | - Ensure pallet truck are place accordingly on vacuum pump base - Ensure load is stable and balanced well on the pallet truck before being transport to new location - Buddy system, one to control the pallet truck steering while the others to push the load - Workers to be alert and keep close lookout on route of travel from point to point - To stop moving and readjust the pallet truck to the centre of gravity of vacuum pump if vacuum pump is not stable while moving | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 25/11/21 |  |
|  |  | Passage way block by material and tools | Trip & fall | - Passage way and working area to be keep clear of material and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 25/11/21 |  |
|  |  | Obstruction to vision while moving | Damage to property if hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert the worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of vacuum pump so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 25/11/21 |  |
|  |  | Noise hazard | Noise-induce deafness | - Look for existing safety signage indicating known noise hazards area - PPE (ear plug) provided and put on when required - Limit the duration of working time when worker are exposed to excessive noise hazard | 3 | 1 | 3 | NIL | 3 | 1 | 3 | Supervisor on site | 25/11/21 |  |