Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 51 Corporation Rd | Welding operation | 1 | Welding of column onto I-Beam |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Welding operation | RA Member: Zailani |  |  |  |
| Location: | 51 Corporation Rd Avitron Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 09/02/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Welding of column onto I-Beam | Welding flash | Damage to eye | - Welding mask with correct grade lens must be worn - Safety glass to be worn by the worker | 3 | 3 | 9 | No other person to be allowed in work area without protection | 3 | 2 | 6 | Supervisor on site | 21-22 /02/22 |  |
|  |  | Spark can cause fire | Bodily Injury | - Ensure Hot Work Permit to be obtained from the host prior to commencement of work - No work will take place in area where the presence of flammable or explosive substances - To remove any combustible materials away from the work area - Use of long sleeve shirt - PPE (glove, welding mask, safety glass) provided and shall be worn by all personnel operating the welding - All necessary fire prevention equipment such as fire blanked/fire extinguishers to be placed near the work area | 4 | 3 | 12 | Regular enforcement on safety rules and regulation among workers | 4 | 2 | 8 | Supervisor on site | 21-22 /02/22 |  |
|  |  | Electric shock | Bodily Injury | - Ensure all equipment is correctly earthed before starting welding - Ensure welding settings are properly check - Ensure cables and hoses are not damage or entangled - Supervisor to ensure all equipment are in good working condition | 4 | 3 | 12 | No unauthorised person to be allowed in work area | 4 | 2 | 8 | Supervisor on site | 21-22 /02/22 |  |
|  |  | Trip & slip | Bodily Injury | - Ground around work area to be free of obstruction - Ground to be check for any grease or liquid spills - Safety shoe to be worn and care taken to prevent slips and trips - Work area to be clear of any obstructions which might cause interruption to welding operation | 3 | 1 | 3 | NIL | 3 | 1 | 3 | Supervisor on site | 21-22 /02/22 |  |