Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | NTU (Simtech Tower Blk) | Lifting / Moving Operation | 1 | Loading Machine onto Pallet Truck or Rollers |  |
|  |  |  | 2 | Transport of Machinery to designated location |  |
|  |  |  | 3 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 4 | Pre-Lift |  |
|  |  |  | 5 | Setting of crane outriggers |  |
|  |  |  | 6 | Lifting Operation |  |

| Department: | Machine Mover | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lifting & Moving of Machine | RA Leader : Zailani |  |  |  |
| Location: | 4 Fusionopolis Way, Kinnesis Building | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 26/08/2016 |  | Name: | Eric Wong |  |
| Last Review Date: | 26/08/2016 |  | Designation: | Director |  |
| Next Review Date | 26/08/2019 |  | Date: | 23/05/18 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Loading machine onto Pallet Truck or Rollers | Stuck by load | Bodily Injury | - Machine to be lower down slowly - To stand by woodblock under machine before inserting rollers/pallet truck - Insert rollers below machine base with care - Use glove | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 24-25/05/ 18 |  |
| 2 | Transportation of machine to designated location | Machine topper | Bodily Injury & Damage to Property | - Ensure rollers/pallet truck are sitting at correct position on the machine base - Ensure machine is stable on the rollers/pallet truck before moving - Workers to be alert and keep close lookout of the movement from point to point on route of travel | 4 | 2 | 8 | Slow and steady transportation speed | 4 | 1 | 4 | Supervisor on site | 24-25/05/ 18 |  |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity pallet truck / rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | 3 | 3 | 9 | Use machine puller or forklift if possible | 3 | 2 | 6 | Supervisor on site | 24-25/05/ 18 |  |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 24-25/05/ 18 |  |
| 3 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury, damage to site property | - Vehicle to be move at slow speed when entering customer premise - Vehicle only to be move into site with the aid of banks man guiding the driver - Vehicle must not be left unattended while engine is running | 3 | 3 | 9 | Assignment of traffic controller | 3 | 2 | 6 | Lorry loader operator | 24-25/05/ 18 |  |
| 4 | Pre-Lifting / Preparation work for lifting | Manual handling heavy equipment / moving traffic | Bodily Injury & pinch point | - PPE (etc, safety helmet, safety shoe, glove,) provided and put on - All lifting equipment are tested by an approved person and has valid certificate - Trained and competent person to check all lifting equipment - Buddy system when handling heavy equipment or gears - Barricade operation area, ensure only relevant personnel is allowed to enter the lifting zone | 3 | 3 | 9 | Lifting crew is pre-briefed before commencement of work | 3 | 2 | 6 | Lifting Supervisor on site | 24-25/05/ 18 |  |
| 5 | Setting of outriggers | Moving parts | Bodily Injury & Damage to Property | - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation and ensure all personnel is not in-way of any parts of the crane - Use steel plate / timber mats for ground support when required - Use of suitable hand glove - Ensure no unauthorised personnel near the lifting zone | 3 | 3 | 9 | Ensure lifting machine on firm and stable ground | 3 | 2 | 6 | Lifting Supervisor on site | 24-25/05/ 18 |  |
| 6 | Lifting Operation | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting Supervisor on site | 24-25/05/ 18 |  |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Riggers appointed and trained to carry out tasks - Ensure rigging of load as per manufacturing manual - Lifting gears tested and approved by competent person - No defective gears allowed | 4 | 2 | 8 | Communication through hand signal by Eric Wong personnel only | 4 | 1 | 4 | Lifting Supervisor on site | 24-25/05/ 18 |  |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keeps hands off loose part of sling or lifting gears - Glove to be provided | 3 | 3 | 9 | Good coordination between workers | 3 | 2 | 6 | Lifting Supervisor on site | 24-25/05/ 18 |  |
|  |  | View obstruction or poor visibility, Load Swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Taglines should be used to control the swing motion - Clear obstruction in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Ensure area of operation clear of unauthorised personnel - PPE (etc, safety shoe, safety helmet, glove) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting Supervisor on site | 24-25/05/ 18 |  |