Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 3 Joo Koon Circle | Lorry loader / overhead crane operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Using lorry crane to pull out case from container onto lorry chassis |  |
|  |  |  | 3 | Uncrate case |  |
|  |  |  | 4 | Using forklift to unload smaller items |  |
|  |  |  | 5 | Using factory overhead crane to hoist machine and assemble parts |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Overhead Crane Operation | RA Member: Zailani |  |  |  |
| Location: | 3 Joo Koon Circle AME International Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 16/09/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 20/09/24 |  |
| 2 | Using lorry loader to pull out case from container onto lorry chassis | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Ensure LM for crane are valid - Ensure sling use to pull out case have valid certificate - To ensure proper command are given to crane operator - To ensure no unauthorised personnel is near the working area - Workers to be alert and keep close lookout to ensure case is being pull out slowly - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Do not force pull the case out of container to minimize any damage if hit side of container | 4 | 1 | 4 | Supervisor on site | 20/09/24 |  |
| 3 | Uncrate case | Falling of wooden plank | Bodily Injury | - P.P.E (safety shoe, safety helmet, glove) provided and put on - Uncrate and dismantle case in the correct sequence - Remove immediately any loose plank from work area - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel near working area | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 20/09/24 |  |
|  |  | Protruding nail on loose timber | Bodily Injury | - P.P.E (safety shoe) provided and put on - Remove immediately any loose timber with protruding nail from work area - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 20/09/24 |  |
| 4 | Using forklift to unload smaller item | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure load weight are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 20/09/24 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Ensure forklift driver is guided by other stuff when required | 4 | 1 | 4 | Supervisor on site | 20/09/24 |  |
| 5 | Use factory overhead crane to hoist machine and assemble parts | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE(safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% on site to supervised the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 20/09/24 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 20/09/24 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Rigger to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong onsite personnel only | 4 | 1 | 4 | Lifting supervisor on site | 20/09/24 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose parts of sling or lifting gears - Ensure proper means of signal are given to overhead crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker re hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 20/09/24 |  |
|  |  | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Appointed and trained lifting supervisor, rigger and signalman on site - Tagline shall be install at both side of load when required to gain control while being hoist up - To clear obstruction and unauthorised personnel in way of lifting path | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Lifting supervisor on site | 20/09/24 |  |