Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | Blk | Mobile Crane Operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Hoisting of lifting platform to level 5 |  |
|  |  |  | 4 | Securing of lifting platform using mobile crane |  |
|  |  |  | 5 | Lifting operation |  |
|  |  |  | 6 | Signalling of load to location |  |
|  |  |  | 7 | Working at height on top platform |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Mobile Crane Operation | RA Member: Zailani |  |  |  |
| Location: | Blk 4 Kaki Bukit Ave 1 #05-01 Moveon Technologies Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date: | 30/06/2026 |  | Date: | 24/01/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle to be moved into site with the aid of banks man guiding the driver to location - To clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 18/10/24 |  |
| 2 | Setting up of crane on site | Crane topple due to outrigger not fully extended | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outrigger (must be able to see all operation procedure - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats as base plate - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator and lifting supervisor to determine the load radius and slewing path of crane to ensure no hitting of nearby building - PPE (safety shoe, safety helmet) provided and put on | 5 | 2 | 10 | Supervisor to ensure no unauthorised personnel or vehicle near moving parts of crane | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
|  |  | Lifting area not cordon off | Bodily Injury | - To demarcate off lifting area with safety cones, red/white tape to ensure no entry to unauthorised personnel - Execute operation during non-peak hours if possible - Total closure of parking/ hoisting area when required | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement of traffic and personnel within the vicinity | 4 | 1 | 4 | Lifting supervisor on site | 18/10/24 |  |
| 3 | Hoisting of lifting platform to level 5 | Improper signal to crane operator | Multiple Injuries & Damage to Property | - Ensure that signalman possesses relevant skill and is fluent in language spoken by crane operator - Use of walkie-talkie to communicate whenever signalman is not visible to crane operator - Ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab - Use strong fastening belt secured front left and right of platform onto building railing or pillar to prevent platform swing outward | 5 | 2 | 10 | Close supervision by lifting supervisor | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
| 4 | Securing of lifting platform using mobile crane | Collision of crane boom | Multiple Injuries & Damage to Property | - Entire lifting crew, such as lifting supervisor, rigger and signalman is trained and to have relevant skill, certificate and experiences required to carry out tasks - Crane operator and lifting supervisor to ensure sufficient turning angle and to confirm no obstruction to crane rotational movement - Ensure platform is tied back to the competent point of platform | 5 | 2 | 10 | Ensure platform is fully rested on the opening slab and stabilise effectively | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
| 5 | Lifting operation | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, try not to jerk or sudden stop the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
|  |  | Overloading | Multiple Injuries & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Multiple Injuries & Damage to Property | - Entire lifting crew, such as lifting supervisor, rigger and signalman is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - To coordinate well with crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site |  |  |
|  |  | Load lifting device gave way | Multiple Injuries & Damage to Property | - To ensure bolts or lifting device connecting lifting device to the machine is properly secured and tighten using spanner or allen key - Ensure bolts specification are comparably to the machine track - To inform supervisor of any loose bolts which is incomparable to the machine track | 5 | 2 | 10 | Eric Wong supervisor to view the lifting device together with machine manufacture personnel on site if needed before any hoisting work | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
| 6 | Signalling of load to location | Load hit structure due to improper signalling | Multiple Injuries & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Use of walkie-talkie to communicate whenever signalman is not visible to crane operator Close supervision by lifting supervisor | 5 | 1 | 5 | Lifting supervisor on site | 18/10/24 |  |
| 7 | Working at height on top platform | Falling from height | Multiple Injuries | - Workers working at height on top platform must have attended WAH course - Self Retracting Lifeline provided and properly secured onto building railing/pillar to act as anchorage point - Supervisor to ensure workers working at height on top platform is fully equip with safety helmet, safety harness and properly secure onto Self Retracting Lifeline while working on top platform at all time - To ensure fall prevention equipment are adequate for all personnel working on top platform - Check all PPE use are in good working condition and free from defects - Supervisor to ensure removable side railing of platform must be in place - Practice good housekeeping | 5 | 2 | 10 | Full supervision and regular remainder by supervisor and to ensure all safety precaution are to be followed and in place No lifting is to be done during bad weather | 5 | 1 | 5 | Supervisor on site | 18/10/24 |  |
|  |  | Platform move or sway | Multiple Injuries & Damage to Property | - Ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secured front left and right of platform onto building railing/pillar to prevent platform swing outward - Platform to be tied back to the competent point of platform using mobile crane | 5 | 2 | 10 | Full supervision and regular reminder by supervisor and to ensure all safety precaution are to be followed and in place | 5 | 1 | 5 | Supervisor on site | 18/10/24 |  |