Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 16 Tuas Ave 7 | Lift up machine to insert I beam | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Jack up machine to 4” high using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lift up machine to insert I beam | RA Member: Zailani |  |  |  |
| Location: | No 16 Tuas Ave 7 ST Kinetics | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 24/01/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topple due to lost of balance | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block to insert below machine - Ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced mover to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 25/01/18 |  |
| 2 | Jack up machine to 4” high using pallet truck | Unstable load when being jack up | Bodily Injury & Damage to Property | - Use the correct capacity pallet truck for the job - Ensure pallet truck are place accordingly on the machine base - Ensure machine load is spread evenly and is stable on the pallet truck - Jack up machine to 4” high at slow speed and to ensure equipment is always stable while on top of the pallet truck - Stop jacking when the height is enough for I beam to be insert under machine bed | 4 | 2 | 8 | Close supervision and regular reminder by supervisor | 4 | 1 | 8 | Supervisor on site | 25/01/18 |  |