Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | No 48 Gul Circle | Moving/Hoisting Machine | 1 | Jack up machine using hydraulic jack |  |
|  |  |  | 2 | Insert rollers |  |
|  |  |  | 3 | Transportation of machine to open yard |  |
|  |  |  | 4 | Site preparation |  |
|  |  |  | 5 | Lifting operation |  |
|  |  |  | 6 | Weather climate |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving / Hoisting machine | RA Leader : Zailani |  |  |  |
| Location: | No 48 Gul Circle OMS Oilfield Service Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2012 |  | Designation: | Director |  |
| Next Review Date | 14/05/2015 |  | Date: | 13/08/2014 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine using manual / hydraulic jack | Machine topper | Bodily Injury & Damage to Property | - Use the correct capacity jack to lift up the machine - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block to insert below machine - Ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced mover to control the jack handling | 4 | 2 | 8 | Supervisor on site |  |  |
| 2 | Insert rollers | Sharp edges of machine | Hand & Finger Injury | - Keep hand and finger clear of sharp edges - Insert rollers to base of machine with care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site |  |  |
| 3 | Transportation of machine to open yard | Rollers move out of position causing machine to topper | Bodily Injury & Damage to Property | - Ensure machine are sitting at correct position on the rollers - Ensure rollers are properly secured to machine base using screw to prevent from rolling out - Ensure machine is stable on the rollers before moving - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 3 | 12 | Slow and steady transportation speed | 4 | 2 | 8 | Supervisor on site |  |  |
|  |  | Excessive weight load | Sprain Back | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | 3 | 3 | 9 | Use machine puller or forklift if possible to assist in pulling or pushing the machine | 3 | 2 | 6 | Supervisor on site |  |  |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | 2 | 3 | 6 | Good coordination | 2 | 2 | 4 | Supervisor on site |  |  |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert the workers controlling the rollers steering if potential hitting of other obstacles | 3 | 3 | 9 | Provision of spotter | 3 | 2 | 6 | Supervisor on site |  |  |
| 4 | Site Preparation | Collapse or topple of crane | Bodily Injury & Damage to Property | - Certificate of testing by an approved person - Operator trained and competent person appointed - Outriggers fully extended - Use steel plate / timber mats for ground support when required - Lifting area cordoned off | 4 | 2 | 8 | Ensure lifting machine is on firm and stable ground | 4 | 2 | 8 | Lifting supervisor on site |  |  |
| 5 | Lifting Operation | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of crane - Load weight should be known | 4 | 3 | 12 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 2 | 8 | Lifting supervisor on site |  |  |
|  |  | Stuck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Riggers appointed and trained to carry out tasks - Ensure rigging of load as per manufacturing manual - Lifting gears tested and approved by competent person - No defective gears allowed | 4 | 3 | 12 | Communication through hand signalling method by Eric Wong personnel only | 4 | 2 | 8 | Lifting supervisor on site |  |  |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arms or Hand Injury | - Keep hands off loose part of sling or lifting gears - Glove to be provided and worn | 3 | 3 | 9 | Good coordination between workers | 3 | 2 | 6 | Lifting supervisor on site |  |  |
|  |  | View obstruction or poor visibility | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Taglines should be use to control the swing motion - Never stand close or under the suspended load and away from possible swing path - Ensure area of operation clear of unauthorised personnel - PPE (etc, safety shoe, safety helmet) provided and put on | 3 | 3 | 9 | Signalman to position himself to be visible to crane operator using hand signalling method or via walkie-talkie | 3 | 2 | 6 | Lifting supervisor on site |  |  |
| 6 | Weather climate | Poor visibility due to heavy rain | Cause unexpected danger in term of workers slipping / strike by lightning | - To stop work immediately if there is heavy rain / lightning strike | 4 | 2 | 8 | Supervisor to ensure no activity during heavy rain | 4 | 2 | 8 | Supervisor on site |  |  |