Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | No 3 Research Link (IMRE Building) – No 21 Nanyang Link (SPMS Level 4) | Shifting out of machinery | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Insert rollers/pallet truck |  |
|  |  |  | 3 | Transportation of machine to loading bay |  |
|  |  |  | 4 | Loading machine onto lorry chassis using forklift |  |
|  |  |  | 5 | Transporting machine out of premise |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving of Machinery | RA Leader : Zailani |  |  |  |
| Location: | From No 3 Research Link (IMRE Building) – No 21 Nanyang Link (SPMS Level 4) | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/06/2015 |  | Name: | Eric Wong |  |
| Last Review Date: | 08/09/2015 |  | Designation: | Director |  |
| Next Review Date | 08/09/2018 |  | Date: | 08/09/2015 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topper | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block to insert below machine - Ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced mover to control the crowbar handling | 4 | 2 | 8 | Supervisor on site | 09/09/2015 |  |
| 2 | Insert rollers/ pallet truck | Sharp edges of machine | Hand & Finger Injury | - Keep hand and finger clear of sharp edges - Insert rollers base of machine with care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 09/09/2015 |  |
| 3 | Transportation of machine to loading bay | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers/pallet truck are place accordingly on machine base - Ensure machine is stable on the rollers/pallet truck before being transport to loading bay - Buddy system-one to control the pallet truck steering while the others to push and hold onto the machine - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow and steady transportation speed | 4 | 2 | 8 | Supervisor on site | 09/09/2015 |  |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | 2 | 3 | 6 | Good coordination | 2 | 2 | 4 | Supervisor on site | 09/09/2015 |  |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert the workers controlling the rollers/pallet truck steering if potential hitting of other obstacles | 3 | 3 | 9 | Provision of spotter | 3 | 2 | 6 | Supervisor on site | 09/09/2015 |  |
| 4 | Loading / unloading machine onto lorry chassis using forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure stability of load before driving off | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 2 | 8 | Supervisor / forklift driver on site | 09/09/2015 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver, if needed, is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller | 4 | 2 | 8 | Supervisor / forklift driver on site | 09/09/2015 |  |
| 5 | Transporting machine out of premise | Unstable / topper of load | Damage to Property | - Load to be properly secured with strapping belt or any other necessary equipment to ensure it does not topper - Drive at slow transportation speed prior to moving out of customer premise | 4 | 2 | 8 | Keep path clear with no objects and personnel obstructing the driveway | 4 | 2 | 8 | Supervisor / driver on site | 09/09/2015 |  |