Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | No 3 Research Link (NUS IMRE Building) | Shifting out of Machinery | 1 | Lift up machine using crowbar |  |
|  |  |  | 2 | Insert rollers/pallet truck |  |
|  |  |  | 3 | Transportation of machine to loading bay |  |
|  |  |  | 4 | Loading machine onto lorry chassis |  |
|  |  |  | 5 | Transporting machine out of premise |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving of Machinery | RA Leader : Zailani |  |  |  |
| Location: | NUS IMRE Building | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 15/06/2015 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Lift up machine using crowbar | Machine topper | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block to insert below machine - Ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced mover to control the crowbar handling | 4 | 2 | 8 | Supervisor on site | 18/06/2015 |  |
| 2 | Insert rollers/ pallet truck | Sharp edges of machine | Hand & Finger Injury | - Keep hand and finger clear of sharp edges - Insert rollers base of machine with care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 18/06/2015 |  |
| 3 | Transportation of machine to loading bay | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers/pallet truck are place accordingly on machine base - Ensure machine is stable on the rollers/pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow and steady transportation speed | 4 | 2 | 8 | Supervisor on site | 18/06/2015 |  |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | 2 | 3 | 6 | Good coordination | 2 | 2 | 4 | Supervisor on site | 18/06/2015 |  |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert the workers controlling the rollers/pallet truck steering if potential hitting of other obstacles | 3 | 3 | 9 | Provision of spotter | 3 | 2 | 6 | Supervisor on site | 18/06/2015 |  |
| 4 | Loading machine onto lorry chassis | Load topper | Bodily Injury & Damage to Property | - Lay out metal plate to cover gap between lorry and loading bay - Push load slowly into lorry chassis - Push load into lorry chassis one at a time - To deploy adequate manpower for the job - PPE (etc… safety shoe, glove) provided and put on | 4 | 2 | 8 | Lorry chassis to be clear of any objects prior to load being push in | 4 | 2 | 8 | Supervisor on site | 18/06/2015 |  |
| 5 | Transporting machine out of premise | Load topper | Damage to Property | - Load to be properly secured with strapping belt or any other necessary equipment to ensure it does not topper - Drive at slow transportation speed prior to moving out of customer premise | 4 | 2 | 8 | Keep path clear with no objects and personnel obstructing the driveway | 4 | 2 | 8 | Supervisor on site | 18/06/2015 |  |