Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | {{location}} | {{process}} |  |  |  |

| Company: | {{company}} | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | {{process}} | RA Member: Zailani |  |  |  |
| Location: | {{location}} | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | {{date}} |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams to be fully extended and resting on steel plate/timber mats as base plate - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | 12/02/2026 |  |
|  |  | Crane overturn due to unstable ground or improper outrigger support | Fatality / Multiple Injuries & Damage to Property | - Ground condition and setup area to be checked before deployment - Outrigger base to rest on steel plate / timber mats - Lorry crane to be set up on firm and level ground - Crane operator to stop work if ground is not suitable | 5 | 2 | 10 | Do not set up crane over drain cover, soft ground or uneven surface without proper support | 5 | 1 | 5 | Lifting supervisor on site | 12/02/2026 |  |
| 3 | Lifting operation | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervise the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 12/02/2026 |  |
|  |  | Load swing or load struck against structure / machine | Multiple Injuries & Damage to Property | - Trial lift to be carried out before actual lifting - Lifting path to be planned and kept free from obstruction - Use tagline where practicable to control the load - Only appointed rigger / signalman to guide the lifting | 4 | 2 | 8 | Stop lifting during strong wind or poor visibility and reposition load slowly into landing area | 4 | 1 | 4 | Lifting supervisor on site | 12/02/2026 |  |
| 4 | Signalling of load | Miscommunication between signalman and crane operator | Multiple Injuries & Damage to Property | - Only appointed signalman to give signal to crane operator - Standard hand signal to be used at all time - Signalman to station at safe position with clear view of load and landing area - No other worker to give conflicting instruction | 4 | 2 | 8 | Use radio communication when line of sight is poor and stop work immediately if signal is not clear | 4 | 1 | 4 | Lifting supervisor on site | 12/02/2026 |  |
| 5 | Unloading of machine using lorry crane | Falling load | Multiple Injuries & Damage to Property | - Machine weight and lifting point to be confirmed before lifting - Proper lifting slings / shackles to be used and checked before use - Trial lift to ensure balance of load - Ensure no unauthorised personnel within the working zone | 5 | 2 | 10 | Set down load on firm and level surface only and keep personnel clear during landing | 5 | 1 | 5 | Lifting supervisor on site | 12/02/2026 |  |
|  |  | Hitting people or property during unloading | Multiple Injuries & Damage to Property | - Operator to lift and slew slowly - Signalman to guide crane movement throughout the operation - Working area to be clear of obstruction - Barricade to be set up around unloading area | 4 | 2 | 8 | Assign one banksman to control pedestrian movement around unloading area | 4 | 1 | 4 | Lifting supervisor on site | 12/02/2026 |  |
| 6 | Insert rollers under machine bed | Finger / hand caught between machine base, roller and flooring | Crush Injury to hand / fingers | - Machine to be raised in stages using suitable jack - Use solid wood block as packing before inserting rollers - Workers to keep hands away from pinch point - Only experienced workers to carry out roller insertion | 4 | 2 | 8 | Use crowbar / pinch bar to adjust roller position and do not place hand under suspended load | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |
|  |  | Machine topple due to loss of balance during insertion of rollers | Multiple Injuries & Damage to Property | - Check centre of gravity before jacking - Jack up machine from one side to another in stages - Use suitable roller size and quantity according to machine weight - Supervisor to ensure area is clear of unauthorised personnel | 5 | 2 | 10 | Keep machine lifting height to minimum required and ensure rollers are aligned evenly before moving | 5 | 1 | 5 | Supervisor on site | 12/02/2026 |  |
| 7 | Transport machine to designated location using rollers | Machine topple or shift out of rollers | Multiple Injuries & Damage to Property | - Workers to be alert when moving machine on rollers - Ensure machine weight is evenly distributed on rollers - Use suitable rollers and steel / metal plate where required for floor protection - Move machine slowly with one appointed person coordinating the movement | 5 | 2 | 10 | Do not push machine abruptly and stop work if roller alignment is out or travel path is uneven | 5 | 1 | 5 | Supervisor on site | 12/02/2026 |  |
|  |  | Foot / hand caught by moving rollers or machine | Crush Injury to hand / foot | - Workers to keep clear from roller travel line - Use pinch bar / pry bar to steer machine - Safety shoe to be worn at all time - Only involved workers to remain in moving area | 4 | 2 | 8 | Brief all workers on escape route and safe standing position before movement starts | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |
|  |  | Damage to floor or obstruction causing sudden stop of machine | Multiple Injuries & Damage to Property | - Travel path to be inspected before movement - Uneven floor / trench / gap to be covered with steel plate or timber packing - Clear all loose materials and obstruction from route | 4 | 2 | 8 | Supervisor to recheck route continuously as machine progresses | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |
| 8 | Unload machine from rollers using hydraulic / hand jack | Machine topple due to lost of balance upon lifted up | Multiple Injuries & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine from one side to another is carried out in stages with the use of solid woodblock to be insert below machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |
|  |  | Finger / hand trapped while removing rollers and packing | Crush Injury to hand / fingers | - Remove rollers only after machine is properly supported by jack and wood block - Use pinch bar to remove roller - Keep hands away from underside of machine - One worker to coordinate the jacking sequence | 4 | 2 | 8 | Do not remove all support at one time; lower machine gradually and evenly | 4 | 1 | 4 | Supervisor on site | 12/02/2026 |  |