| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) |  |
| Process/ Location: | Hoist and detach machine from base using overhead crane 12 Tuas Ave 20 - Flowserve |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 12 July 2016 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Hoist and detach machine from base | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance to weight of machine - No defective gears allowed - Never stand close or under the suspended load - PPE (etc…safety helmet, safety shoe, glove) provided and put on | Major | Remote | Medium | While moving or slewing the crane, try not to jerk the load | Lifting supervisor on site |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | Major | Remote | Medium | Ensure load, capacity of overhead crane use are within limits, when in doubt, always use a higher capacity crane | Lifting Supervisor on site |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out task - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgement - Ensure load are rig at it centre of gravity | Major | Remote | Medium | Good coordination between workers | Lifting Supervisor on site |
|  |  | Hands caught by or in between loose part of sling or lifting gears | Arm of Hand Injury | - Ensure signalman give proper means of signal to crane operator - Keep hands off loose part of sling or lifting gears - Use glove | Moderate | Remote | Low |  | Lifting supervisor on site |
|  |  | Movement of load | Collision with Personnel & Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Use tag line for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Ensure movement is guided - Never stand close or under the suspended load and away from possible swing path | Major | Remote | Medium | Communication through hand signalling method by Eric Wong personnel only | Lifting Supervisor on site |