| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Lifting/position machine using overhead crane No 10 Gul Circle (OMS Oilfield Services PL) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 23 June 2014 |  |  |  |  |
|  |  | Last Review Date: | # 23 June 2014 | Next Review Date: | 22 June 2017 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Using overhead crane to hoist down machine to ground floor | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Ensure no lifting beyond the SWL of overhead crane - Load weight should be known - Ensure area of operation are clear of unauthorised personnel - Never stand close or under the suspended load | Major | Remote | Medium | While slewing and lowering, try not to jerk the load | Lifting supervisor on site |
|  |  | Movement of load | Collision with People & Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Tagline, if needed should be use to control the swing motion | Major | Remote | Medium | Communication through hand signalling method by Eric Wong personnel only | Supervisor on site |
| 2 | Insert rollers | Sharp edges of machine | Hand & Finger Injury | - Keep hand and fingers clear of sharp edges - Insert rollers to machine base with care - Use of hand glove | Minor | Occasional | Low |  | Supervisor on site |
| 3 | Transportation of machine to designated location | Rollers move out of position causing machine to topper | Bodily Injury & Damage to Property | - Ensure machine are sitting at correct position on the rollers - Ensure rollers are properly secured to machine base using screw to prevent from rolling out - Ensure machine is stable on the rollers before moving - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | Major | Remote | Medium | Slow and steady transportation speed | Supervisor on site |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity pallet truck / rollers to reduce the effort of each man when pushing the machine - Deploy adequate personnel | Moderate | Occasional | Medium | Use machine puller or forklift if possible | Supervisor on site |
|  |  | Passage way block by equipment and tools | Trip & Fall | - Passage way working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil / water spill must be clean up immediately | Moderate | Occasional | Medium | Good coordination | Supervisor on site |
|  |  | Obstruction to vision | Bodily Injury & Damage to Property | - To keep close watch on route of travel - Ensure movement is guided - To alert the worker controlling the rollers steering if potential hitting of other obstacles | Moderate | Occasional | Medium | Provision of spotter | Supervisor on site |