| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Member) Zaharin (Team Member) Wong Yew Fei (Team Member) |  |
| Process/ Location: | Crating of wooden cases (normal or vacuum pack) No 15 Tuas View Place (John Crane Singapore Pte Ltd) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 23 Jan 2017 |  |  |  |  |
|  |  | Last Review Date: | # 15 May 2015 | Next Review Date: | 15 May 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Manual handling of equipment | Improper lifting poster | Back injury | - Adopt proper technique when carrying out task - Buddy system when handling heavy equipment - Ensure there are enough manpower for the job | Moderate | Remote | Low |  | Supervisor on site |
|  |  | Accidental dropping of equipment | Foot injury | - PPE (etc.. safety shoe) issued and worn by all workers - If needed, use glove for firm grip on equipment - Ensure no unauthorised personnel within the working area | Moderate | Occasional | Medium | Use of mechanical lifting aids (etc.. manual lifter, forklift) if needed | Supervisor on site |
| 2 | Drilling and screw with power tools | Continuous noise generated | Noise induce hearing lost | -PPE (etc…ear plug) provided and put on | Moderate | Occasional | Medium | Rotating shift of workers | Supervisor on site |
| 3 | Use of pneumatic tools | Faulty parts of pneumatic tools | Bodily injury due to tool accidentally disconnected | - Regular inspection of hose connection - Ensure hose connection for pneumatic tools are fastened securely to pipe outlet and equipment | Moderate | Remote | Low |  | Supervisor on site |
|  |  | Trip by cable | Bodily Injury | - Ensure cables are place properly on the ground, tied off if needed - Proper housekeeping to be maintained during work | Minor | Remote | Low |  | Supervisor on site |
| 4 | Working on ladder | Fall from ladder | Injury to workers | - Use “A” frame ladder to work on which is unreachable from ground - Ladder must be fully extended and stand on level ground - Ensure buddy system-one to hold onto the ladder while the other is working on the ladder | Moderate | Remote | Low |  | Supervisor on site |
| 5 | Use of nail gun | Mishandling of nail gun | Bodily Injury to workers or nearby personnel | - Only trained and competent personnel are allowed to use tools that are issued to them - Understudy personnel need the supervision of a trained personnel - Ensure possible routes of projectiles are clear of unauthorised personnel in case of miss fired | Moderate | Remote | Low |  | Supervisor on site |
| 6 | Hot vacuum sealer sealing the vacuum bag | Heat from sealer | Burnt | - Follow proper working procedure of vacuum sealer - Only trained and competent personnel are allowed to use the tools that are issued to them | Moderate | Remote | Low |  | Supervisor on site |