| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Lay down Tester to Horizontal position using forklift NUS – Architecture Drive (School of Design & Environment) |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 15 Oct 2015 |  |  |  |  |
|  |  | Last Review Date: | # 15 Oct 2015 | Next Review Date: | 15 Oct 2018 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Drive in forklift into premise | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure working space is clear of unauthorised personnel and any obstruction - Ensure forklift driver, if needed is guided by other staff - Drive at slow transportation speed | Major | Remote | Medium | Assignment of traffic controller | Forklift Driver / Supervisor on site |
| 2 | Working at height to secured wire/webbing sling | Fall from height | Bodily injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ladder must be fully extended and stand on ground level - Ensure buddy system-one to hold the ladder while the other one is working on the ladder | Major | Remote | Medium | Use of safety harness attached to a strong anchorage point | Supervisor on site |
| 3 | Insufficient working space | Hitting personnel or property | Bodily injury, damage to property | - To ensure there are enough working space to lay down Tester to horizontal position - To keep working path clear with no objects and unauthorised personnel obstructing the pathway | Major | Remote | Medium |  | Supervisor on site |
| 4 | Lay down Tester to horizontal position | Failing of lifting equipment | Bodily injury, damage to property | - Riggers to ensure lifting gears use has valid certificate and in compliance to weight of load - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure load are lower down at slow and steady speed - Ensure no personnel is standing close or under the load | Major | Remote | Medium | Use extension fork if needed to give greater stability | Forklift Driver / Supervisor on site |