| Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company: | Eric Wong Machinery Tptn Pte Ltd | Conducted by: (Names, designations) (Date) |  | Kevin Wong (Team Leader) Zailani (Team Leader) Zaharin (Team Member) |  |
| Process/ Location: | Dismantle & lay down 1 unit Hoist Structure using forklift/scissor lifter No 3 Tuas Loop |  |  |  |  |
| Approved by: (Name, designation) (Date) | Eric Wong (Director) 05 Jan 2016 |  |  |  |  |
|  |  | Last Review Date: | # 05 Jan 2016 | Next Review Date: | 05 Jan 2019 |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation |  |  |  | 3. Risk Control |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a. | 1b. | 1c. | 1d. | 2a. | 2b. | 2c. | 2d. | 3a. | 3b. |
| No. | Work Activity | Hazard | Possible Accident / Ill Health & Persons-at-Risk | Existing Risk Control (if any) | Severity | Likelihood | Risk Level | Additional Risk Control | Action Officer, Designation (Follow-up date) |
| 1 | Drive in forklift / scissor lifter into factory premise | Hitting people or property | Bodily injury, damage to site property | - Only certified and competent forklift / scissor lifter operator are allowed to operate the equipment - Ensure working space is clear of unauthorised personnel and any obstruction - Ensure forklift driver, if needed is guided by other staff - Drive at slow transportation speed | Major | Remote | Medium | Assignment of traffic controller | Supervisor on site |
| 2 | Working at height to secured webbing sling / loosen nuts | Fall from height | Bodily injury | - Provision of scissor lifter as working platform - Ensure only certified and competent personnel are allowed to operate the scissor lifter - Worker working at height to put on safety harness, safety helmet - Ensure safety harness use are in good working condition, properly strapped on and hook to strong anchorage point at all time | Major | Remote | Medium | Full supervision by supervisor at all time | Supervisor on site |
| 3 | Lay down structure using forklift | Falling load | Bodily injury, damage to property | - Only certified and competent forklift operator are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure load are lower down at slow and steady speed - Ensure no personnel is standing close or under the load | Major | Remote | Medium | Full supervision by supervisor at all time | Supervisor on site |
| 4 | Insufficient working space | Hitting personnel or property | Bodily injury, damage to property | - To ensure there is enough working space to dismantle and lay down structure - To keep working area clear with no objects or unauthorised personnel obstructing the pathway | Major | Remote | Medium | Full supervision by supervisor at all time | Supervisor on site |